EmbedSim  CodeGen

Architecture & Design Guide

MISRA C:2012  ·  ISO 26262 ASIL-D  ·  AURIX TC3xx  ·  ARM Cortex-M

Revision 3.0 — March 2026

First verified end-to-end run on AURIX TC38x TASKING toolchain

# 1  Design Philosophy

EmbedSim is built on one principle that governs every architectural decision:

Python is the specification. C is the artefact. Never the reverse.

This means the simulation runs first, in Python, with full observability — plots, topology diagrams, signal histories. Code generation is the last step, not the first. The generated C is a faithful translation of the verified Python model, not a hand-crafted implementation that the Python tries to replicate.

The consequence is that there are no hand-written C strings anywhere in simulation files. Every string that appears in generated C — function names, struct types, argument lists — is derived automatically from block metadata. A simulation file that contains a C string is carrying a generator bug in disguise.

# 2  The Three-Layer Block Pipeline

Every EmbedSim block has three layers. The layers share the same algorithm — only the execution environment differs.

| Layer | Technology | Role |
| --- | --- | --- |
| 1 — Python | NumPy / pure Python | Specification and verification. Runs during simulation. Full observability. |
| 2 — Cython | .pyx + C extension | Performance path. Called via compute_c(). Same algorithm, compiled. |
| 3 — Generated C | EmbedSim_step.c/.h | Production target. Emitted by StepGenerator. Runs on MCU bare-metal. |

The float32 contract binds all three layers: every signal array crossing a layer boundary must be numpy.float32 on the Python side and real32_T on the C side. Violating this causes a silent failure — Cython does not auto-cast dtypes, so a float64 array passed to a float[::1] memoryview silently leaves the internal buffer at zero. The motor hums but does not rotate. All duties read exactly 0.5.

# 3  The CodeGen Boundary

The CodeGen boundary is defined by two marker blocks: CodeGenStart and CodeGenEnd. Everything between them is code-generated. Everything outside runs in Python simulation only.

This boundary is a design decision, not a technical constraint. You choose where to place it based on what logic belongs on the MCU. The boundary can be moved without changing any block implementation — only the wiring changes.

## 3.1  EmbedSim_Input_T — What Crosses the Boundary Inward

The input struct contains exactly the signals that the MCU integration layer is responsible for computing and providing. For the DB42S02 open-loop controller, the boundary is placed after SVPWMPackBlock — so the integration layer owns the Cartesian-to-polar conversion:

typedef struct {

real32_T magnitude;   /* sqrtf(v_alpha^2 + v_beta^2), clipped to 0.95 */

real32_T angle_rad;   /* atan2f(v_beta, v_alpha), wrapped to [0, 2pi)  */

} EmbedSim_Input_T;

When the boundary moves — for example, when closed-loop FOC is added — the input struct grows to include sensor feedback:

/* Closed-loop FOC — future boundary */

typedef struct {

real32_T omega_m;   /* measured rotor speed [rad/s] */

real32_T i_a;       /* phase A current [A]          */

real32_T i_b;       /* phase B current [A]          */

} EmbedSim_Input_T;

The struct is generated automatically from cg_start.iter_signals(). No manual editing.

## 3.2  EmbedSim_Output_T — What Crosses the Boundary Outward

The output struct contains the signals the MCU writes to actuators. For the SVPWM controller:

typedef struct {

real32_T ta;      /* Phase A duty cycle [0, 1] */

real32_T tb;      /* Phase B duty cycle [0, 1] */

real32_T tc;      /* Phase C duty cycle [0, 1] */

uint8_T  sector;  /* Active SVPWM sector 0..5  */

} EmbedSim_Output_T;

Note that sector is uint8_T, not real32_T. The generator knows this because SVPWMBlock declares C_OUTPUT_TYPES = {"sector": "uint8_T"}. The type system flows from block metadata to generated struct — no manual intervention.

## 3.3  The ISR Integration Pattern

The generated API is Simulink-equivalent but with explicit I/O and no hidden global state:

/* Simulink */  Model_step(RT_MODEL *M);          /* dt and I/O buried in RT_MODEL */

/* EmbedSim */  EmbedSim_Step(dt, in*, out*);    /* dt and I/O explicit */

The dt parameter is explicit by design. This means the same generated function can be called from ISRs at different rates, passed a measured dt instead of a compile-time constant, or exercised in unit tests at any sample rate — without recompiling.

void FOC_ISR(void) {

EmbedSim_Input_T  in;

EmbedSim_Output_T out;

/* Integration layer owns magnitude/angle computation */

in.magnitude = sqrtf(v_alpha*v_alpha + v_beta*v_beta);

if (in.magnitude > 0.95f) { in.magnitude = 0.95f; }

in.angle_rad = atan2f(v_beta, v_alpha);

if (in.angle_rad < 0.0f) { in.angle_rad += SVM_2PI_F; }

EmbedSim_Step(EMBEDSIM_DT, &in, &out);

/* Write to GTM ATOM compare registers */

ATOM0_CH0_CM0 = (uint32_t)(out.ta * period);

ATOM0_CH2_CM0 = (uint32_t)(out.tb * period);

ATOM0_CH4_CM0 = (uint32_t)(out.tc * period);

/* out.sector available for diagnostics */

}

# 4  Block Metadata — How Blocks Declare Their C Contract

Every block declares its C contract through class attributes. The StepGenerator reads these at code-generation time. No C strings ever appear in simulation files.

## 4.1  Standard Attributes

| Attribute | Type | Purpose |
| --- | --- | --- |
| PYX_FILE | str | Path to .pyx wrapper — triggers PYXInspector auto-population |
| C_SOURCES | list | .c files the block depends on |
| C_HEADERS | list | .h files to #include in generated code (lowercased automatically) |
| NUM_INPUTS | int | 0 for source blocks, N for normal blocks |
| OUTPUT_SIZE | int | Number of scalar outputs in the y[] buffer |
| C_INIT_ARGS | list | Instance attribute names forwarded as typed literals to the Init call |
| C_CUSTOM_EMIT | str | Escape hatch: verbatim C fragment for non-standard ABI only |

## 4.2  Boundary Control Attributes

These attributes give blocks control over what appears in the generated input/output structs. They are the mechanism by which a block exposes a named, typed subset of its signals rather than a raw array.

| Attribute | Purpose |
| --- | --- |
| OUTPUT_NAMES | Named fields to expose in EmbedSim_Output_T, in order |
| OUTPUT_KEEP | Indices into y[] corresponding to each OUTPUT_NAMES entry |
| C_OUTPUT_TYPES | Dict mapping field name to C type. Defaults to real32_T. Use for uint8_T sector etc. |
| INPUT_NAMES | Named fields to expose in EmbedSim_Input_T, in order |
| INPUT_KEEP | Indices into the upstream output corresponding to each INPUT_NAMES entry |
| C_INPUT_TYPES | Dict mapping input field name to C type. Defaults to real32_T. |

Example — SVPWMPackBlock exposes only magnitude and angle_rad to the CodeGen boundary, silently dropping Vdc at index 2:

# Set on the instance in db42s02_openloop_fmu.py

svpwm_pack.INPUT_NAMES = ["magnitude", "angle_rad"]

svpwm_pack.INPUT_KEEP  = [0, 1]   # Vdc at index 2 is integration-layer constant

Example — SVPWMBlock exposes all four outputs including sector as uint8_T:

# Declared in svpwm_block.py

OUTPUT_NAMES   = ["ta", "tb", "tc", "sector"]

OUTPUT_KEEP    = [0, 1, 2, 3]

C_OUTPUT_TYPES = {"sector": "uint8_T"}

INPUT_NAMES / OUTPUT_NAMES live on the block class or instance — never in the simulation file. The simulation file contains zero CodeGen metadata.

# 5  StepGenerator — How C is Emitted

StepGenerator performs a DFS backwards from cg_end to cg_start to collect all blocks in the bounded region. It then emits two files: EmbedSim_step.h (types and prototypes) and EmbedSim_step.c (state variables, Init, Step).

## 5.1  _emit_block() Decision Tree

For each block in topological order, the generator applies this decision tree:

- C_CUSTOM_EMIT set → emit the C fragment verbatim. Skip all auto-generation. Use only for non-standard ABI (e.g. matrix-pointer outputs).

- IS_UNIT_DELAY = True → emit a memcpy from the shared z⁻¹ register. No step function needed.

- NUM_INPUTS == 0 → source block pattern: real32_T y[N]; Step(&state, dt, y);  — no u[] array.

- step_func set → normal block pattern: build u[] from upstream y[] buffers, then call Step.

- C_SOURCES set but no step_func → emit a TODO comment. Generator needs more metadata.

- None of the above → emit a Python-only comment. Block has no C implementation.

Every use of C_CUSTOM_EMIT for a standard block is a generator bug in disguise. The correct fix is to improve _emit_block() so it handles the pattern natively.

## 5.2  Type Flow — From Python to C

The type of each field in the generated structs flows automatically from block metadata:

Python block attribute          →  Generated C struct field

──────────────────────────────────────────────────────────

OUTPUT_NAMES = ["ta", "sector"] →  real32_T ta;

OUTPUT_KEEP  = [0, 3]           →  uint8_T  sector;

C_OUTPUT_TYPES = {"sector":     →  (determined by C_OUTPUT_TYPES)

"uint8_T"}

The pack section in EmbedSim_Step() applies the correct cast:

out->ta     = y_svpwm[0];

out->tb     = y_svpwm[1];

out->tc     = y_svpwm[2];

out->sector = (uint8_T)y_svpwm[3];   /* cast driven by C_OUTPUT_TYPES */

## 5.3  Init Function — Typed Literal Arguments

Stateful blocks declare C_INIT_ARGS as a list of instance attribute names. StepGenerator reads the live values at code-generation time and emits typed float literals with the f suffix:

C_INIT_ARGS = ["omega_target", "ramp_time"]

/* Generated */

SpeedRamp_Init(&speed_ref_state, 41.88790205f, 0.15000000f);

When the CodeGen boundary is moved so that no stateful blocks remain inside the region, EmbedSim_Init() is emitted as an empty function with a comment. This is correct and intentional — the DB42S02 open-loop controller with the Revision 3.0 boundary has no stateful blocks in the generated region.

# 6  PYXInspector — Automatic Metadata Population

PYXInspector parses .pyx wrapper files as text (regex, no import or execution) and auto-populates step_func, state_struct, init_func, NUM_INPUTS, and OUTPUT_SIZE on VectorBlock subclasses at class-definition time via Python's __init_subclass__ hook.

The result is that block definitions require no hand-written C strings:

class SpeedRampBlock(VectorBlock):

PYX_FILE    = str(_C_SRC / "motor_utility_blocks_wrapper.pyx")

C_SOURCES   = ["motor_utility_blocks.c"]

C_HEADERS   = ["motor_utility_blocks.h"]

NUM_INPUTS  = 0      # source block

OUTPUT_SIZE = 1

C_INIT_ARGS = ["omega_target", "ramp_time"]

# step_func, state_struct, init_func — auto-populated by PYXInspector

PYXInspector classifies C functions by keyword in the uppercase function name: INIT → init_func, STEP or COMPUTE or UPDATE → step_func. Auto-populated attributes are only written if not already set — a subclass can override any attribute by declaring it explicitly.

# 7  Extensibility — Adding New Blocks

The architecture is designed so that adding a new code-generated block requires touching exactly two files: the block definition and the .pyx wrapper. The simulation file and code_generator.py require no changes.

- Write the C implementation — .c / .h with the standard Step(&state, u, dt, y) signature.

- Write the Cython wrapper — .pyx with set_inputs(), compute(), get_outputs(). Use float[::1] throughout.

- Build the extension — python setup_*.py build_ext --inplace. Delete stale .pyd / .so before rebuilding.

- Define the Python block — subclass VectorBlock. Declare PYX_FILE, C_SOURCES, C_HEADERS, NUM_INPUTS, OUTPUT_SIZE, C_INIT_ARGS. PYXInspector fills the rest.

- Wire into the simulation — use >> operator. Place between cg_start and cg_end to include in code generation.

For blocks with non-standard C ABI (e.g. coordinate transforms that return outputs via pointer arguments rather than a flat array), use C_CUSTOM_EMIT as an escape hatch. The custom fragment is emitted verbatim inside EmbedSim_Step() and is responsible for declaring its own y_<name>[] buffer so the pack section can read it.

# 8  Moving the CodeGen Boundary

The most important architectural operation in EmbedSim is moving the CodeGen boundary. Because all logic lives in block metadata, the boundary can be repositioned by changing only the simulation wiring — no block implementations change.

## 8.1  Revision History — DB42S02 Boundary Positions

| Rev | cg_start position | EmbedSim_Input_T |
| --- | --- | --- |
| 2.0 | Before SpeedRampBlock | Empty (_reserved byte — C99 compliance) |
| 3.0 | After SVPWMPackBlock | { magnitude, angle_rad } |
| Future | After Clarke transform | { v_alpha, v_beta } |
| Future | After current sensors | { omega_m, i_a, i_b } — closed-loop FOC |

The table shows a progression: as more of the signal chain is absorbed into the MCU integration layer, the CodeGen region shrinks to its essential kernel. The final closed-loop position will contain the full FOC algorithm — PI regulators, Park/Clarke transforms, and SVPWM — with sensor readings as inputs.

## 8.2  Rule — What Belongs Inside the Boundary

The CodeGen region should contain the control algorithm — the computation that must run deterministically at the ISR rate on the MCU. It should not contain:

- Physical constants injected at runtime (V_dc, T_load) — these belong in Init args or integration layer

- Signal conditioning that the MCU application already performs (Clarke transform, sqrtf, atan2f)

- Plant models — FMU blocks are always outside the boundary

If a block inside the CodeGen region reads from a constant that never changes at runtime, it belongs in C_INIT_ARGS, not EmbedSim_Input_T.

# 9  Commissioning Record — Bugs Fixed (March 2026)

The following issues were found and resolved during the first verified end-to-end run on the AURIX TC38x TASKING toolchain.

| # | Symptom | Root cause | Fix |
| --- | --- | --- | --- |
| 1 | NaN on all duties | EmbedSim_Init() not called — v_dc=0 → 1/0=Inf → 0×Inf=NaN | Call EmbedSim_Init() once before first EmbedSim_Step() |
| 2 | All duties = 0.5 exactly | double[::1] in .pyx set_inputs() — silently zeroes float32 inputs | Change to float[::1] in all .pyx wrappers |
| 3 | Motor hums, no rotation | np.float64 in compute_c() — still mismatched after .pyx fix | Change all compute_c() buffer allocations to np.float32 |
| 4 | Motor hums (stale build) | Stale .pyd — setuptools skipped rebuild after .pyx change | Delete .pyd/.c/build/, rebuild from scratch |
| 5 | Compile error: redeclared variable | y_speed_ref[1] declared twice: unpack section AND _emit_block() | Unpack section now skips source blocks (NUM_INPUTS==0) |
| 6 | TASKING E210: not a member name | EmbedSim_Input_T had svpwm_pack[3] array instead of named scalars | INPUT_NAMES/INPUT_KEEP mechanism added to StepGenerator._in_sigs() |
| 7 | sector absent from Output_T | OUTPUT_NAMES excluded sector; no C_OUTPUT_TYPES declared | Add sector to OUTPUT_NAMES/KEEP; add C_OUTPUT_TYPES={sector:uint8_T} |
| 8 | ta/tb/tc wrong scale | Q31 integer cast directly to real32_T — value ~2.1e9 not [0,1] | Divide by Q31_ONE in C_CUSTOM_EMIT: svm_duty.ta / Q31_ONE |

# 10  Glossary

| Term | Definition |
| --- | --- |
| C_CUSTOM_EMIT | Verbatim C fragment emitted by StepGenerator. Escape hatch for non-standard ABI only. |
| C_OUTPUT_TYPES | Dict mapping output field name to C type. Overrides default real32_T. |
| CodeGenEnd | Marker block at the output boundary of the generated region. |
| CodeGenStart | Marker block at the input boundary of the generated region. |
| INPUT_NAMES / KEEP | Block attributes that expose a named scalar subset of upstream output to EmbedSim_Input_T. |
| OUTPUT_NAMES / KEEP | Block attributes that expose a named scalar subset of y[] to EmbedSim_Output_T. |
| PYXInspector | Parses .pyx files to auto-populate step_func, state_struct, init_func on block classes. |
| real32_T | typedef float real32_T — single-precision IEEE 754. Used throughout generated C. |
| StepGenerator | Emits EmbedSim_step.c / .h from the CodeGenStart/End bounded region. |
| float[::1] | Cython C-contiguous float32 memoryview. Mandatory in all set_inputs() signatures. |
| ATOM | Advanced Timer Output Module — GTM sub-module. 24-bit, ARU-connected. Used for motor PWM on AURIX. |
| SVPWM | Space Vector PWM. Modulates three-phase inverter duty cycles from a voltage vector in αβ space. |