EmbedSim  Technical Reference

Open-Source Embedded Control Simulation & Code Generation Framework

AURIX TriCore  ·  ARM Cortex-M  ·  MISRA C:2012  ·  ISO 26262 ASIL-D

Revision 3.0 — March 2026

# 1  Overview

EmbedSim is a Python-first simulation and C code-generation framework for embedded motor control. It provides:

- Block-diagram simulation engine — VectorBlock, VectorSignal, >> wiring operator, DFS topological sort, RK4/Euler ODE solvers

- FMU co-simulation — Modelica plant models via FMPy (FMI 2.0)

- Cython/C extension pipeline — single-precision performance-critical blocks compiled to .pyd/.so

- StepGenerator — automatic emission of MISRA C:2012-compliant EmbedSim_step.c / .h

- TopologyPrinter — interactive HTML signal-flow diagrams with CodeGen boundary visualisation

Design principle: Python is the specification. C is the artefact. Never the reverse.

# 2  Architecture

## 2.1  Block Pipeline

Every EmbedSim block follows a three-layer architecture:

| Layer | Technology | Role |
| --- | --- | --- |
| 1 — Python | NumPy | Specification and simulation. compute_py(). Full observability. |
| 2 — Cython | .pyx extension | Performance path. compute_c(). Same algorithm, compiled to .pyd/.so. |
| 3 — C | EmbedSim_step.c | Production MCU target. Emitted by StepGenerator. MISRA C:2012. |

## 2.2  The float32 Contract

All signal arrays crossing any layer boundary must be numpy.float32 on the Python side and real32_T (typedef float) on the C side. This contract applies to:

- set_inputs() method signatures in all .pyx files — use float[::1], never double[::1]

- All buffer allocations in compute_c() — use np.zeros(N, dtype=np.float32), never np.float64

- All generated C arrays — real32_T throughout, no double

Violating this contract causes a silent failure: Cython does not auto-cast dtypes. A float64 array passed to a float[::1] memoryview silently leaves the internal buffer at zero. Every block with inputs sees u = [0, 0, ...] every step. All duty cycles read exactly 0.5. The motor hums but does not rotate.

## 2.3  Directory Structure

embedsim_project/

├── embedsim/

│   ├── code_generator.py      StepGenerator, CodeGenStart, CodeGenEnd

│   ├── core_blocks.py         VectorBlock, VectorSignal

│   └── simulation_engine.py   EmbedSim runner, DFS sort

├── fs_electrical_machines/

│   ├── motor_utility_blocks.py

│   ├── coordinate_transform_blocks.py

│   ├── svpwm_block.py

│   ├── pyx_inspector.py

│   └── c_src/

│       ├── motor_utility_blocks.c/.h

│       ├── svpwm.c/.h

│       └── *_wrapper.pyx

└── embedsim_gen/

├── EmbedSim_step.c        Auto-generated — DO NOT EDIT

└── EmbedSim_step.h        Auto-generated — DO NOT EDIT

# 3  VectorBlock — Base Class

Every EmbedSim block subclasses VectorBlock. The key class attributes for CodeGen are:

| Attribute | Type | Purpose |
| --- | --- | --- |
| PYX_FILE | str | Path to .pyx — triggers PYXInspector auto-population |
| C_SOURCES | list | .c files required by StepGenerator |
| C_HEADERS | list | .h files to #include (auto-lowercased by StepGenerator) |
| NUM_INPUTS | int | 0 for source blocks (no upstream input), N otherwise |
| OUTPUT_SIZE | int | Number of scalar outputs in y[] |
| C_INIT_ARGS | list | Instance attribute names forwarded as typed literals to Init() |
| C_CUSTOM_EMIT | str | Verbatim C fragment. Escape hatch for non-standard ABI only. |
| OUTPUT_NAMES | list | Named fields to expose in EmbedSim_Output_T |
| OUTPUT_KEEP | list | Indices into y[] for each OUTPUT_NAMES entry |
| C_OUTPUT_TYPES | dict | C type per output field. Defaults to real32_T. |
| INPUT_NAMES | list | Named fields to expose in EmbedSim_Input_T |
| INPUT_KEEP | list | Indices into upstream y[] for each INPUT_NAMES entry |

# 4  PYXInspector

## 4.1  Purpose

PYXInspector parses .pyx wrapper files as text (regex, no import or execution) and auto-populates step_func, state_struct, init_func, NUM_INPUTS, and OUTPUT_SIZE on VectorBlock subclasses at class-definition time via Python's __init_subclass__ hook.

## 4.2  Classification Rules

PYXInspector classifies C functions by keyword in the uppercase function name:

- INIT in name → init_func

- STEP or COMPUTE or UPDATE in name → step_func

- RESET in name → reset_func

Auto-populated attributes are only written if not already set — a subclass can override any attribute by declaring it explicitly in the class body.

## 4.3  Manual Override

class SpeedRampBlock(VectorBlock):

PYX_FILE    = str(_C_SRC / "motor_utility_blocks_wrapper.pyx")

NUM_INPUTS  = 0      # explicitly set — PYXInspector will not overwrite

OUTPUT_SIZE = 1      # explicitly set

C_INIT_ARGS = ["omega_target", "ramp_time"]

# step_func, state_struct, init_func — auto-populated from .pyx

# 5  StepGenerator

## 5.1  Generated Output Structure

EmbedSim_step.h contains: EMBEDSIM_DT macro, EmbedSim_Input_T typedef, EmbedSim_Output_T typedef, extern declarations for Init() and Step(), and #include directives for all block headers.

EmbedSim_step.c contains: #include directives, static state struct declarations (MISRA Rule 8.7 internal linkage), EmbedSim_Init() calling all block Init functions with typed literal arguments, and EmbedSim_Step(dt, in*, out*) calling all blocks in topological order.

## 5.2  _emit_block() Decision Tree

- C_CUSTOM_EMIT set → emit verbatim, skip all auto-generation

- IS_UNIT_DELAY = True → emit memcpy from z⁻¹ shared register

- NUM_INPUTS == 0 → source block: real32_T y[N]; Step(&state, dt, y);

- step_func set → normal: build u[] from upstream, call Step(&state, u, dt, y)

- C_SOURCES but no step_func → emit TODO comment

- Neither → emit Python-only comment

## 5.3  Critical Rules

- Call EmbedSim_Init() before the first EmbedSim_Step(). Failure causes uninitialised state — v_dc=0 → NaN cascade.

- Pass dt_hz explicitly. Default is 0.0 which emits #define EMBEDSIM_DT (0.0f).

- C_HEADERS entries are auto-lowercased. StepGenerator calls .lower() — use lowercase consistently.

- C_CUSTOM_EMIT is an escape hatch, not a workaround. Every use for a standard block is a generator bug in disguise.

# 6  Cython Wrapper Pattern

## 6.1  Standard Structure

# cython: language_level=3

# cython: boundscheck=False

# cython: wraparound=False

cdef extern from "motor_utility_blocks.h":

ctypedef struct SpeedRamp_T:

float ramp_value

void SpeedRamp_Init(SpeedRamp_T *s, float omega_target, float ramp_time) nogil

void SpeedRamp_Step(SpeedRamp_T *s, float dt, float *y) nogil

cdef class SpeedRampWrapper:

cdef SpeedRamp_T _state

cdef float[1] _y

cpdef void set_inputs(self, float[::1] u): pass   # source — no inputs

cpdef void compute(self, float dt):

with nogil: SpeedRamp_Step(&self._state, dt, self._y)

cpdef cnp.ndarray get_outputs(self):

cdef cnp.ndarray y = np.empty(1, dtype=np.float32)

y[0] = self._y[0]

return y

Use float[::1] in set_inputs(), never double[::1]. Cython will not raise an error on dtype mismatch — it silently leaves the buffer at zero.

## 6.2  Build and Rebuild

cd fs_electrical_machines/c_src

# Clean rebuild — always delete stale artefacts first

del motor_utility_blocks_wrapper.pyd   # Windows

del motor_utility_blocks_wrapper.c

rmdir /s /q build

python setup_motor_utility_blocks.py build_ext --inplace

Setuptools uses file timestamps to decide whether to recompile. A stale .pyd will silently take precedence over updated source. When in doubt, delete everything and rebuild from scratch.

# 7  Known Issues and Resolutions — DB42S02 Commissioning

| # | Symptom | Root cause | Fix |
| --- | --- | --- | --- |
| 1 | NaN on all duties | EmbedSim_Init() not called — v_dc=0 → 1/0=Inf → 0×Inf=NaN | Call EmbedSim_Init() once in state machine one-shot guard |
| 2 | All duties = 0.5 | double[::1] in .pyx set_inputs() silently zeroes float32 inputs | Change to float[::1] in all .pyx wrappers |
| 3 | 0.5 after .pyx fix | compute_c() still allocated np.float64 input buffers | Change all compute_c() buffers to np.float32 |
| 4 | 0.5 after float32 fix | Stale .pyd — setuptools skipped rebuild after .pyx change | Delete .pyd/.c/build/, full clean rebuild |
| 5 | Compile: redeclared var | y_speed_ref[1] declared in unpack section AND _emit_block() | Unpack section skips source blocks (NUM_INPUTS==0) |
| 6 | TASKING E210 member | EmbedSim_Input_T had svpwm_pack[3] instead of named scalars | INPUT_NAMES/INPUT_KEEP added to StepGenerator._in_sigs() |
| 7 | sector missing | OUTPUT_NAMES excluded sector; no C_OUTPUT_TYPES | Add sector to OUTPUT_NAMES/KEEP; C_OUTPUT_TYPES={sector:uint8_T} |
| 8 | ta/tb/tc ~2.1e9 | Q31 int32 cast directly to real32_T without dividing by Q31_ONE | Divide svm_duty.ta/tb/tc by Q31_ONE in C_CUSTOM_EMIT |

# 8  AURIX Integration

## 8.1  GTM ATOM Channel Assignment

| Signal | Module | Channel | Phase |
| --- | --- | --- | --- |
| ta | ATOM0 | CH0 | A |
| tb | ATOM0 | CH2 | B |
| tc | ATOM0 | CH4 | C |

ATOM (Advanced Timer Output Module) is required for motor-control PWM: 24-bit resolution, CMU clock, ARU interconnect. TOM (16-bit, no ARU) is not used for motor PWM on AURIX TC3xx.

## 8.2  State Machine Integration — Revision 3.0

static int embedsim_init_done = 0;

case StateMachine_focOpenLoop:

if (embedsim_init_done == 0) {

EmbedSim_Init();

embedsim_init_done = 1;

}

{

EmbedSim_Input_T  in;

EmbedSim_Output_T out;

/* Integration layer: Cartesian -> polar */

in.magnitude = sqrtf(v_alpha*v_alpha + v_beta*v_beta);

if (in.magnitude > 0.95f) { in.magnitude = 0.95f; }

in.angle_rad = atan2f(v_beta, v_alpha);

if (in.angle_rad < 0.0f) { in.angle_rad += SVM_2PI_F; }

EmbedSim_Step(EMBEDSIM_DT, &in, &out);

Ifx_TimerValue period =

motorCtrl->inverter.pwm3PhaseOutput.timer.base.period;

motorCtrl->inverter.pwm3PhaseOutput.pwmOnTimes[0] =

(Ifx_TimerValue)(out.ta * (float32)period);

motorCtrl->inverter.pwm3PhaseOutput.pwmOnTimes[1] =

(Ifx_TimerValue)(out.tb * (float32)period);

motorCtrl->inverter.pwm3PhaseOutput.pwmOnTimes[2] =

(Ifx_TimerValue)(out.tc * (float32)period);

/* out.sector available for diagnostic logging */

}

Pmsmfoc_doPwmSvmUpdate(&motorCtrl->inverter);

break;

# 9  Motor Parameters — NANOTEC DB42S02

| Parameter | Value | Description |
| --- | --- | --- |
| R | 0.19 Ω | Stator phase resistance |
| Ld = Lq | 0.125 mH | Surface PMSM — isotropic inductance |
| λ_pm | 0.0014 Wb | Permanent magnet flux linkage |
| J | 2.4×10⁻⁶ kg·m² | Rotor moment of inertia |
| p | 4 | Pole pairs |
| V_dc | 17 V | DC bus voltage |
| Rated speed | 8000 RPM | Open-loop V/f reference |
| V/f ratio | V_phase_peak / ω_e_rated | Scalar control law constant |

# 10  Glossary

| Term | Definition |
| --- | --- |
| ATOM | Advanced Timer Output Module — GTM sub-module, 24-bit, CMU input, ARU-connected |
| ARU | Action Request Unit — GTM interconnect for zero-CPU signal routing |
| CodeGen boundary | The region between CodeGenStart and CodeGenEnd that StepGenerator translates to C |
| DEFAULT_DTYPE | numpy.float32 — all EmbedSim signal arrays use this dtype |
| FMU | Functional Mock-up Unit — FMI 2.0 plant model compiled from Modelica |
| FOC | Field-Oriented Control — dq-frame current regulation |
| GTM | Generic Timer Module — AURIX TC3xx PWM and timer subsystem |
| MISRA C | Motor Industry Software Reliability Association C coding standard |
| PMSM | Permanent Magnet Synchronous Motor |
| PYXInspector | Parses .pyx files to auto-populate VectorBlock CodeGen attributes |
| real32_T | typedef float real32_T — single-precision IEEE 754, from Sys_Types.h |
| StepGenerator | Emits EmbedSim_step.c / .h from the CodeGenStart/End bounded region |
| SVPWM | Space Vector PWM — αβ voltage modulation. Sector 1 centred on +α axis at ±30°. |
| TOM | Timer Output Module — GTM sub-module, 16-bit, no ARU. Not used for motor PWM. |
| V/f control | Scalar open-loop speed control — voltage proportional to frequency |
| VectorBlock | EmbedSim base class for all signal-processing blocks |
| float[::1] | Cython C-contiguous float32 memoryview — mandatory in all set_inputs() |