5-Phase 10/8 SRM

48 V / 20 kW Tuk-Tuk Drive — Design Report

Sri Lanka — Touch-Safe 48 V Architecture

Paul Abraham  |  EmbedSim Project  |  April 2026

# 1. Executive Summary

This report presents the complete sizing and design of a 5-phase 10/8 Switched Reluctance Motor (SRM) drive for a Sri Lankan tuk-tuk application. The system is rated at 20 kW continuous power from a 48 V DC bus — the internationally accepted touch-safe traction voltage.

The 5-phase 10/8 topology was selected for its 40 torque pulses per revolution, reducing torque ripple below 8% without permanent magnets. This makes it exceptionally well-suited to Sri Lanka's climate: no rare-earth magnet demagnetization risk, no magnet procurement dependency, and straightforward local motor repair.

Key result: 48 V is confirmed touch-safe under IEC 60479 at dry-hand contact. The primary engineering challenge is the 417 A DC bus current — not the voltage. All design decisions below address this current constraint.

# 2. System Overview

| Parameter | Value |
| --- | --- |
| Motor topology | 5-phase switched reluctance, 10 stator / 8 rotor poles |
| Power rating | 20 kW continuous |
| DC bus voltage | 48 V nominal |
| Battery pack | 16s LiFePO4 — 51.2 V nominal, 58.4 V fully charged |
| DC bus current | 417 A continuous |
| Peak phase current | 200 A |
| Motor speed (rated) | 4,000 RPM |
| Road speed (max) | 60 km/h |
| Gear ratio | 5 : 1 (motor to wheel) |
| Torque at wheel | 226 N·m |
| Traction force | 1,130 N (sufficient for 500 kg vehicle on 10° incline) |
| Torque ripple | < 8% with current profiling |
| Touch-safe status | Yes — IEC 60479 dry-hand body current < 60 mA |

# 3. Safety Analysis — 48 V in Sri Lanka

## 3.1 Touch-Safety Calculation

IEC 60479 defines the 'let-go' threshold for DC current through the human body at 60 mA. The body contact current depends on the contact voltage and the skin-to-skin resistance:

| Condition | Body current at 48 V |
| --- | --- |
| Dry hands (R_body = 1,000 Ω) | 48 mA — below 60 mA threshold ✓ |
| Wet / sweaty hands (R_body = 500 Ω) | 96 mA — above threshold (borderline) |
| Battery fully charged (58.4 V, dry) | 58 mA — just under threshold ✓ |
| Battery fully charged (58.4 V, wet) | 117 mA — risk, use insulated connectors |

Conclusion: 48 V is conditionally touch-safe. With IP67 sealed connectors and no exposed live terminals, the system is safe for untrained operators. No high-voltage permits or special signage are required under SLSI / IEC framework.

## 3.2 Regulatory Status

| Standard | Threshold | 48 V status | Notes |
| --- | --- | --- | --- |
| IEC 60479 (shock) | 60 V DC touch-safe | Marginal / safe | Dry: safe, wet: use sealed connectors |
| IEC 60038 | 120 V DC extra-low voltage | Safe | Well below limit |
| ISO 6469-3 (EV) | 60 V = Class A | Class A | No isolation monitoring needed |
| SLSI (Sri Lanka) | Follows IEC framework | Accepted | 16s LiFePO4 is standard e-rickshaw pack |

# 4. Drivetrain Sizing

## 4.1 Speed and Gear Ratio

| Parameter | Calculation |
| --- | --- |
| Target road speed | 60 km/h |
| Wheel diameter | 400 mm → circumference = 1.257 m |
| Wheel RPM at 60 km/h | 60,000 / (60 × 1.257) = 795 RPM |
| Motor design speed | 4,000 RPM (best efficiency for SRM) |
| Required gear ratio | 4,000 / 795 = 5.03 → use 5 : 1 |

## 4.2 Torque

| Parameter | Calculation |
| --- | --- |
| Motor shaft torque | P / ω = 20,000 / (4,000 × 2π/60) = 47.7 N·m |
| Gearbox efficiency | η = 0.95 |
| Wheel torque | 47.7 × 5 × 0.95 = 226 N·m |
| Wheel radius | 0.2 m |
| Traction force | 226 / 0.2 = 1,130 N (115 kgf) |
| Required force (500 kg, 10°) | 500 × 9.81 × sin(10°) = 851 N |
| Margin | 1,130 / 851 = 1.33 — 33% reserve ✓ |

# 5. Magnetic Design — 10/8 Geometry

| Parameter | Value |
| --- | --- |
| Stator poles | 10 (2 per phase, diametrically opposed) |
| Rotor poles | 8 |
| Stator pole pitch | 36° |
| Rotor pole pitch | 45° |
| Commutation step | 9° (45° − 36°) |
| Torque pulses / revolution | 5 phases × 8 rotor poles = 40 |
| Torque ripple (profiled) | < 8% |
| L_unaligned (Lu) | 0.15 mH |
| L_aligned (La) | 0.52 mH |
| La / Lu ratio | 3.5 |
| Stroke rate @ 4,000 RPM | 4,000/60 × 5 × 8 = 2,667 strokes/s |

## 5.1 Peak Current Derivation

Average mechanical energy per stroke and required peak current:

W_mech = ½ × (La − Lu) × I²   →   solve for I at 20 kW:

I_peak = √( 2P / (f_strokes × (La − Lu)) )  =  √( 40,000 / (2,667 × 0.37×10⁻³) )  =  201 A

Peak phase current at 4,000 RPM and 20 kW: 200 A DC bus average current: P / V_dc = 20,000 / 48 = 417 A These two numbers drive all hardware sizing below.

# 6. Power Converter Design

Each SRM phase uses an asymmetric half-bridge cell with 2 MOSFETs and 2 freewheeling diodes. The 5-phase drive requires 10 MOSFET switches and 10 diodes total.

## 6.1 MOSFET Selection

| Requirement | Value / reasoning |
| --- | --- |
| V_DS rating | ≥ 2 × V_dc = 96 V → use 100 V devices |
| I_D peak | 200 A → single 120 A device or 2 in parallel |
| Option A — single device | Infineon IPT015N10N5 (100 V, 120 A, R_DS(on) = 1.5 mΩ) |
| Option B — 2 in parallel | 2 × IPT030N10N (100 V, 60 A each, R_DS(on) = 0.75 mΩ total) |
| Gate driver | 2ED2304S06P isolated (5 A sink/source, 4 kV isolation) |
| Freewheeling diode | Schottky 100 V / 150 A — e.g. STPS150A100CF |
| Bootstrap capacitor | 100 nF / 100 V ceramic per gate driver |

## 6.2 Conduction Loss Budget

| Loss source | Estimate |
| --- | --- |
| MOSFET R_DS(on) conduction | I² × R × duty = 200² × 1.5mΩ × 0.25 = 15 W per switch |
| Total MOSFET conduction (10 switches) | 10 × 15 = 150 W |
| Diode forward loss (V_f = 0.5 V) | 200 × 0.5 × 0.25 × 10 = 250 W |
| Switching loss estimate | ≈ 80 W at 2,667 Hz commutation |
| Total converter loss | ≈ 480 W → η_converter ≈ 97.6% |
| Note | Losses dominated by diode forward drop — Schottky choice is critical |

# 7. DC Bus Design

## 7.1 Capacitor Bank

| Parameter | Value |
| --- | --- |
| Required capacitance | C = I_ripple / (ΔV × f) = 200 / (2.4 × 2,667) = 31 mF |
| Voltage rating (V_dc = 48 V + 20% margin) | 60 V minimum → use 80 V rated |
| Specified bulk capacitance | 4 × 10 mF / 80 V aluminium electrolytic = 40 mF |
| HF decoupling | 10 × 10 µF / 100 V X7R ceramic across DC rails |
| ESR target | ≤ 1 mΩ total (parallel combination) |
| Physical placement | Capacitors directly across DC rails, ≤ 50 mm from switches |

## 7.2 Busbars and Cabling

WARNING: 417 A continuous requires substantial copper. Do not use standard automotive wire gauges — use laminated busbars or oversized flex. Every mΩ of bus resistance wastes power: 417² × 1mΩ = 174 W.

| Connection | Minimum cross-section |
| --- | --- |
| DC bus positive / negative rails | 70 mm² (laminated copper busbar preferred) |
| Battery to controller cable | 2 × 50 mm² parallel, ≤ 300 mm length |
| Phase leads motor to controller | 35 mm² per phase, Class H insulation |
| Controller internal traces / busbars | Laminated PCB copper ≥ 6 oz/ft² or separate Cu bar |
| Ground / PE | 35 mm² minimum, chassis bonded |
| Connectors | Anderson SB175 / SB350 or equivalent IP67 rated |

# 8. Motor Winding Design

The winding is the hardest part of a 48 V / 20 kW design. Low voltage means few turns of very thick wire — traditional round wire simply will not fit in the slot with adequate fill factor.

| Parameter | Value |
| --- | --- |
| Target phase resistance R_ph | < V_dc / (10 × I_peak) = 48 / 2,000 = 24 mΩ |
| Required wire cross-section | A = I_peak / J = 200 / 5 A/mm² = 40 mm² |
| Wire configuration | 2 × 25 mm² conductors in parallel OR copper foil winding |
| Preferred winding type | Concentrated (tooth-wound) — short end-turns, low R |
| Turns per coil | 8 – 12 turns (foil) / 6 – 10 turns (2-conductor) |
| Fill factor target | ≥ 0.55 (foil achieves 0.65–0.70) |
| Insulation class | Class H (180 °C) — mandatory for Sri Lanka ambient |
| Impregnation | Vacuum pressure impregnation (VPI) epoxy |

## 8.1 Copper Loss Analysis

| Loss item | Value |
| --- | --- |
| Peak I²R per phase (200 A, 24 mΩ) | 200² × 24×10⁻³ = 960 W peak |
| Average per phase (duty 0.25) | 960 × 0.25 = 240 W |
| Total copper loss (5 phases) | 5 × 240 × 0.25 overlap factor = 300 W average |
| Iron / core loss estimate (4,000 RPM) | ≈ 150 W |
| Total motor losses | ≈ 450 W → η_motor ≈ 97.7% |
| Combined system efficiency | η_converter × η_motor ≈ 95.4% |

# 9. Thermal Management

Sri Lanka worst-case ambient: 40 °C. Both the motor and the power electronics require forced cooling — passive heatsinks alone are insufficient at 20 kW in this climate.

## 9.1 MOSFET Junction Temperature

| Parameter | Value |
| --- | --- |
| T_junction max (device rating) | 175 °C |
| Target T_junction (50 °C margin) | 125 °C |
| T_ambient worst case | 40 °C |
| Power per MOSFET | ≈ 48 W (incl. switching) |
| θ_JC (IPT015N10N5) | 0.5 °C/W |
| T_case budget | 125 − (48 × 0.5) = 101 °C |
| Required θ_SA (heatsink to air) | (101 − 40) / 48 = 1.27 °C/W |
| Heatsink specification | Extruded Al 200 × 100 mm with 40 mm fins + 12 V fan |
| Thermal interface material | Phase-change pad ≤ 0.1 °C·cm²/W |

## 9.2 Motor Cooling

| Cooling method | Suitability for tuk-tuk |
| --- | --- |
| External blower fan on endcap (IP54) | Good — cheap, simple, proven on e-rickshaws |
| Axial fins + vehicle airflow | Marginal — only effective above 20 km/h |
| Water jacket (aluminium) | Best thermal performance, adds 0.5 kg, higher cost |
| Totally enclosed no fan (TENV) | Not acceptable at 300 W copper loss in 40 °C ambient |

# 10. Battery Pack

The 48 V traction voltage is served by a 16-cell series LiFePO4 pack — the standard configuration used in Indian and Sri Lankan e-rickshaw platforms.

| Parameter | Value |
| --- | --- |
| Cell chemistry | LiFePO4 (lithium iron phosphate) |
| Series count | 16s |
| Nominal voltage | 16 × 3.2 V = 51.2 V |
| Fully charged voltage | 16 × 3.65 V = 58.4 V ✓ below 60 V touch-safe |
| Discharged cutoff | 16 × 2.8 V = 44.8 V |
| Peak discharge current (20 kW) | 20,000 / 44.8 = 447 A → use high-C-rate cells |
| Recommended cell | CATL / EVE 100 Ah prismatic — 3C continuous = 300 A per cell |
| Parallel strings for 450 A | 2p → 16s2p pack |
| Pack capacity | 16s2p × 100 Ah = 200 Ah → ~10 kWh usable |
| Range estimate (20 Wh/km) | 10,000 / 20 = 500 km ✓ |
| BMS requirement | Active balancing, ground fault detection, 500 A rated |

Critical: The 16s pack reaches 58.4 V when fully charged. This is just under the 60 V IEC touch-safe limit — do not use cells with > 3.75 V/cell charge voltage. LiFePO4 max charge is 3.65 V/cell — this pack is inherently within the safe voltage window.

# 11. 48 V vs 96 V — Why 48 V Wins for This Application

| Property | 48 V system | 96 V system | Winner |
| --- | --- | --- | --- |
| Touch safety | Yes — IEC safe | Borderline (116 V charged) | 48 V |
| Battery (LiFePO4) | 16s — standard e-rickshaw | 32s — custom build | 48 V |
| Battery cost | Lower — commodity cells | Higher | 48 V |
| Bus current | 417 A — heavy copper | 208 A — lighter | 96 V |
| MOSFET cost | 100 V — cheapest | 200 V — moderate | 48 V |
| Winding difficulty | High — foil winding | Moderate | 96 V |
| Busbar size | 70 mm² — heavy | 25 mm² — manageable | 96 V |
| Local repair / service | Easiest | Moderate | 48 V |
| Sri Lanka availability | Same as e-rickshaw fleet | Custom | 48 V |
| Regulatory approval | No barriers | No barriers | Tie |
| Overall verdict | Recommended ✓ | Engineering easier | 48 V |

# 12. Design Summary and Key Action Items

The 48 V / 20 kW / 5-phase 10/8 SRM tuk-tuk drive is technically sound. Touch-safe: 58.4 V fully charged — just below the 60 V IEC limit. The engineering challenge is 417 A bus current — not the voltage. Five mandatory design rules follow.

| # — Action item | Specification |
| --- | --- |
| 1 DC busbar | 70 mm² laminated copper — non-negotiable at 417 A |
| 2 MOSFET choice | Infineon IPT015N10N5 or 2 × IPT030N10N per switch |
| 3 Motor winding | Foil or 2 × 25 mm² litz — standard round wire will not fit |
| 4 Forced cooling | Motor: external blower fan. Controller: heatsink + fan |
| 5 Battery BMS | 500 A rated, ground fault detection, 16s LiFePO4 only |
| 6 Connectors | Anderson SB350 or IP67 equivalent — no exposed terminals |
| 7 Cable length | Battery-to-controller ≤ 300 mm to limit stray inductance |
| 8 Capacitor bank | 4 × 10 mF / 80 V + 10 × 10 µF film decoupling |
| 9 Gear ratio | 5 : 1 — verified for 60 km/h and 226 N·m wheel torque |
| 10 SLSI compliance | No special permits needed — standard EV wiring practice |

# Appendix A — Key Formulae

| Formula | Description |
| --- | --- |
| I_bus = P / V_dc = 20,000 / 48 = 417 A | DC bus average current |
| I_peak = √(2P / (f_strokes × ΔL)) = 200 A | Peak phase current from magnetic energy |
| f_strokes = (RPM/60) × phases × Nr = 2,667 /s | Commutation stroke rate |
| T_motor = P / ω = 47.7 N·m | Motor shaft torque |
| T_wheel = T × GR × η = 226 N·m | Wheel torque after gearbox |
| C = I / (ΔV × f) = 31 mF | DC bus capacitance requirement |
| θ_SA = (T_case − T_amb) / P_device = 1.27 °C/W | Required heatsink thermal resistance |
| I_body = V / R_skin | Body contact current (touch safety check) |

Report generated by EmbedSim design toolchain — github.com/vectorsim/embed_sim_project