Deep Architecture Reference

Python  --  Cython  --  C Code Generation  --  FMU / Modelica

Revision 3.0  --  March 2026  --  MISRA C:2012 -- ISO 26262 ASIL-D

“Python is the specification.  C is the artefact.  Never the reverse.”

What this document covers: The complete internal mechanics of EmbedSim -- how the simulation engine traverses block graphs, how C names are derived from Python metadata, how types flow from Python attributes to generated struct fields, how Cython bridges the Python and C worlds, how FMU plants are wrapped and connected, and how three concrete examples illuminate each mechanism. Read the code alongside this document; the code is the proof.

Part 1   Architecture: The Four-Layer Stack

## 1.1  The Central Principle

EmbedSim is built around one inversion of the usual embedded workflow. Normally you write C, flash it, probe it with a debugger, and reverse-engineer what the algorithm is doing. EmbedSim inverts this: you write the algorithm in Python first, simulate it to full numerical correctness, and then have the framework translate it to C automatically.

The consequence is that the Python simulation and the MCU firmware are always identical in behaviour. Not approximately identical -- literally the same algorithm, translated mechanically. If the Python simulation is correct, the MCU firmware is correct. The only remaining risk is in the translation mechanics themselves, which is why those mechanics are documented here in full.

## 1.2  Four Layers, Four Roles

The stack has exactly four layers. Each has a single, well-defined role that does not overlap with any other layer.

| Layer | Technology | Single responsibility | What you hand-write |
| --- | --- | --- | --- |
| 1 -- Python | NumPy / pure Python | Specify and verify the algorithm | compute_py() method body |
| 2 -- Cython | .pyx + C extension | Compile the same algorithm for performance | cdef extern declarations, cpdef wrappers |
| 3 -- Generated C | MISRA StepGenerator | Translate verified Python to MCU production code | Nothing: it is emitted automatically |
| 4 -- FMU / Modelica | OpenModelica + FMPy | Provide physics-correct plant models | Modelica equations (.mo file) |

The fundamental invariant:  No information flows from Layer 3 back to Layer 1. The C code is always derived from the Python model, never the reverse. A simulation file that contains a C type name, a C function name, or a C string literal is carrying a code-generator bug in disguise. Every name in generated C is derived programmatically from Python block metadata.

## 1.3  How Information Flows Between Layers

Understanding EmbedSim requires understanding how data and metadata flow between layers. There are two distinct flows: the runtime signal flow (what happens during simulation) and the code-generation metadata flow (what happens when you call generate_step()).

### Runtime signal flow (simulation time)

EmbedSim.run()

for each step k:

_compute_all_blocks(t, dt)

for each block in topological order:

input_values = [upstream_block.output for upstream in block.inputs]

block.compute(t, dt, input_values)

-> routes to compute_c() or compute_py()

-> updates block.output  (a VectorSignal wrapping a np.float32 array)

Signal type at every boundary:

VectorSignal.value  = np.float32 array   <- Python side

float[::1] memoryview                    <- Cython side (same buffer, no copy)

real32_T*  pointer                       <- C side (same buffer, no copy)

### Code-generation metadata flow (generate_step() time)

cg_end.generate_step(cg_start, output_dir, dt_hz)

StepGenerator.__init__()

DFS backward from cg_end to cg_start

collect ordered block list

StepGenerator.generate_header()

collect INPUT_NAMES from cg_start upstream blocks -> EmbedSim_Input_T fields

collect OUTPUT_NAMES from cg_end upstream blocks  -> EmbedSim_Output_T fields

#include all C_HEADERS from all blocks in region

emit EMBEDSIM_DT = 1/dt_hz

StepGenerator.generate_source()

emit static state declarations: block.state_struct  block.name + "_state"

emit EmbedSim_Init(): call block.init_func for each stateful block

emit EmbedSim_Step(): call _emit_block() for each block in order

## 1.4  Naming Convention -- How Every C Name is Derived

Every C identifier in generated code is derived mechanically from Python attributes. There are no magic strings, no hardcoded names, no manual mappings. Understanding the derivation rules is the key to understanding what the generated code will look like.

| Generated C identifier | Derived from | Example |
| --- | --- | --- |
| State variable name | block.name + "_state" | dfc -> dfc_state |
| State struct type | block.state_struct | DFC_State_T |
| Init function call | block.init_func + "(&" + name + "_state, ...)" | DFC_Controller_Init(&dfc_state, ...) |
| Step function call | block.step_func + "(&" + name + "_state, ...)" | DFC_Controller_Step(&dfc_state, ...) |
| y[] buffer name | "y_" + block.name | y_dfc, y_svpwm_pack |
| u[] buffer name | "u_" + block.name | u_dfc, u_svpwm |
| Input struct field | INPUT_NAMES[i] | in->omega_ref_mech, in->theta_m |
| Output struct field | OUTPUT_NAMES[i] | out->ta, out->sector |
| Init literal argument | str(getattr(block, C_INIT_ARGS[i])) | 41.88790205f (from block.omega_target) |
| Header #include | C_HEADERS[i].lower() | #include "embed_sim_dfc_controller.h" |
| Timestep macro | "EMBEDSIM_DT" value = 1/dt_hz as f-literal | #define EMBEDSIM_DT (0.00005000f) |

The lowercase rule:  StepGenerator calls .lower() on every C_HEADERS entry. This enforces consistent lowercase include paths regardless of how the Python string was written. A block that declares C_HEADERS = ["Embed_Sim_DFC_Controller.H"] will generate #include "embed_sim_dfc_controller.h" -- always matching the actual filesystem convention.

Part 2   Simulation Engine: DFS, Algebraic Loops, and Execution Order

## 2.1  The Block Graph

Every EmbedSim block subclasses VectorBlock. Blocks are connected by the >> operator, which appends the right-hand block to the left-hand block's successors list and appends the left-hand block to the right-hand block's inputs list. At this stage no computation happens -- the graph is a pure Python object graph.

sin_src >> loop_sum >> loop_gain >> loop_out

# Equivalent to:

# sin_src.successors.append(loop_sum);  loop_sum.inputs.append(sin_src)

# loop_sum.successors.append(loop_gain); loop_gain.inputs.append(loop_sum)

# loop_gain.successors.append(loop_out); loop_out.inputs.append(loop_gain)

# >> is associative and supports fan-out:

dfc >> svpwm_pack >> svpwm >> cg_end   # chain

dfc >> svpwm_pack                       # fan-out (two >> calls to same target)

## 2.2  Topological Sort -- Two-Pass DFS

When EmbedSim.__init__() is called, it runs traverse_blocks_from_sinks_with_loops() to produce a flat ordered list blocks_order. This list is the execution order for every simulation step. The algorithm is a depth-first search (DFS) from every sink block, run in two passes.

### Pass 1: find_loop_breakers()

Walk the entire graph from every sink, following block.inputs recursively. Collect every block whose class inherits from LoopBreaker into the set loop_breakers. Identification is O(1) via block.is_loop_breaker flag -- no isinstance() on every edge.

### Pass 2: dfs() -- post-order with loop handling

def dfs(block, visiting, visited, loop_breakers, blocks_order):

if block in visited:

return

if block in visiting:

# back-edge: check if it is a LoopBreaker

if block not in loop_breakers:

raise ValueError(f"Algebraic loop detected at: {block.name}")

# LoopBreaker: insert NOW, stop recursion (back-edge is cut)

if block not in visited:

blocks_order.append(block)

visited.add(block)

return

visiting.add(block)

if block not in loop_breakers:

for inp in block.inputs:

dfs(inp, visiting, visited, loop_breakers, blocks_order)

# post-order: append after all inputs

blocks_order.append(block)

visiting.discard(block)

visited.add(block)

The counter-intuitive result: a VectorDelay block is appended to blocks_order BEFORE its upstream source. In the closed-loop DFC example, motor_delay is placed first in the execution list -- before ctrl_packer, before dfc, before motor. This is correct: motor_delay outputs motor(k-1), which was stored during the previous step. It does not need motor to have run at step k.

## 2.3  Runtime Execution: _compute_all_blocks()

Every simulation step calls _compute_all_blocks(t, dt). This iterates blocks_order exactly once, left to right:

def _compute_all_blocks(self, t, dt):

# Step A: pre-initialise LoopBreakers whose output is None (t=0 only)

for block in self.blocks_order:

if isinstance(block, LoopBreaker) and block.output is None:

block.output = block.get_loop_breaking_output()

# VectorDelay: returns VectorSignal(initial)

# Step B: forward pass

for block in self.blocks_order:

input_values = []

for inp in block.inputs:

if inp.output is not None:

input_values.append(inp.output)

elif isinstance(block, LoopBreaker):

input_values.append(VectorSignal([0.0]))  # silent: by design

else:

self.logger.warning(f"zero-fallback: {block.name} <- {inp.name}")

input_values.append(VectorSignal([0.0]))

block.compute(t, dt, input_values)   # updates block.output

Why zero-fallback exists:  At t=0, motor_delay runs first. Its upstream (motor) has output=None because motor has not yet run this step. The engine substitutes [0.0] and continues. motor_delay ignores this substitute -- it returns last_output=initial regardless of what the current input is. The warning fires only at t=0 and disappears at t=1 because motor.output is then set from the previous step.

## 2.4  ODE Integration

After _compute_all_blocks() completes for a step, the engine advances stateful dynamic blocks (integrators, state-space models) through the chosen ODE solver. EmbedSim supports three solvers:

| Solver | Order | Cost | Use case |
| --- | --- | --- | --- |
| ODESolver.EULER | 1 | 1x _compute_all_blocks | Fastest; acceptable for AURIX hardware-matched simulations at 20 kHz |
| ODESolver.HEUN | 2 | 2x _compute_all_blocks | Recommended default: predictor-corrector, 2nd-order accuracy |
| ODESolver.RK4 | 4 | 4x _compute_all_blocks | Highest accuracy; 4x cost; triggers 4x zero-fallback warnings at t=0 |

VectorDelay is not an ODE state. Its state update (latching current input as last_output) happens inside compute_py() at Step B. The ODE integrator sees VectorDelay as a stateless block. This is correct: the z^-1 operation is a sample-and-hold, not an integral.

## 2.5  VectorSignal -- The Signal Carrier

VectorSignal is a thin wrapper around a np.float32 array with an optional name tag. It is the type of every block.output and every element of input_values. The float32 dtype is enforced at construction: passing a float64 array quietly truncates to float32. The DEFAULT_DTYPE = np.float32 constant is imported throughout the codebase to avoid any accidental float64 construction.

class VectorSignal:

def __init__(self, value, name=""):

# value is always stored as float32

if isinstance(value, np.ndarray):

self.value = value.astype(np.float32)

else:

self.value = np.array(value, dtype=np.float32)

self.name = name

# Usage at block output:

self.output = VectorSignal(

np.array([v_alpha, v_beta], dtype=DEFAULT_DTYPE),

self.name

)

Part 3   VectorBlock: Metadata, Routing, and the C Contract

## 3.1  VectorBlock Base Class

Every EmbedSim processing block subclasses VectorBlock. The base class provides the >> wiring operator, the use_c_backend routing flag, the compute() dispatcher, and all the class-level attributes that the code generator reads.

class VectorBlock:

# ---- CodeGen attributes (read by StepGenerator) ----

PYX_FILE       = None    # str: path to .pyx wrapper (triggers PYXInspector)

C_SOURCES      = []      # list of .c files

C_HEADERS      = []      # list of .h files (auto-lowercased by StepGenerator)

NUM_INPUTS     = 1       # 0 for source blocks (no upstream)

OUTPUT_SIZE    = 1       # number of scalar outputs in y[]

C_INIT_ARGS    = []      # instance attribute names -> typed Init() literals

C_CUSTOM_EMIT  = None    # str: verbatim C emitted for non-standard ABI

C_CODEGEN_EXCLUDE = False# True -> never enters StepGenerator

# ---- Boundary struct attributes ----

INPUT_NAMES    = []      # field names for EmbedSim_Input_T

INPUT_KEEP     = []      # indices into upstream y[] for each INPUT_NAMES entry

C_INPUT_TYPES  = {}      # dict: field -> C type (default: real32_T)

OUTPUT_NAMES   = []      # field names for EmbedSim_Output_T

OUTPUT_KEEP    = []      # indices into y[] for each OUTPUT_NAMES entry

C_OUTPUT_TYPES = {}      # dict: field -> C type (default: real32_T)

# ---- Auto-populated by PYXInspector (if PYX_FILE is set) ----

step_func    = None      # str: C step function name

state_struct = None      # str: C state struct type name

init_func    = None      # str: C init function name

# ---- Topology ----

TOPO_CATEGORY  = "block" # "plant", "utility", "source", "sink", "codegen"

output_label   = ""      # displayed in topology printer

## 3.2  The compute() Dispatcher

The compute() method is the single call point for the simulation engine. Every VectorBlock subclass that has a C backend implements both compute_py() and compute_c(). The dispatcher routes based on use_c_backend and _wrapper is not None:

# In DFControllerBlock (and all blocks with C backends):

def compute(self, t, dt, input_values=None):

if self.use_c_backend and self._wrapper is not None:

return self.compute_c(t, dt, input_values)

return self.compute_py(t, dt, input_values)

# The C backend path: compute_c()

def compute_c(self, t, dt, input_values=None):

# 1. Unpack upstream VectorSignal into a flat np.float32 array

inputs    = np.zeros(5, dtype=np.float32)

inputs[0] = float(u[0])   # omega_ref_mech [rad/s]

inputs[1] = float(u[1])   # theta_m [rad]

inputs[2] = float(u[2])   # ia [A]

inputs[3] = float(u[3])   # ib [A]

inputs[4] = float(u[4])   # ic [A]

# 2. Call through Cython wrapper -- zero copy (float[::1] shares buffer)

self._wrapper.set_inputs(inputs)    # float[::1] memoryview

self._wrapper.compute(float(dt))    # calls DFC_Controller_Step() in C

outputs = self._wrapper.get_outputs() # returns np.float32 array

# 3. Wrap in VectorSignal and store

self.output = VectorSignal(outputs, self.name)

return self.output

## 3.3  PYXInspector -- Automatic Metadata Population

PYXInspector is a static analysis tool that reads .pyx wrapper files as plain text using regular expressions. It runs at class-definition time via Python's __init_subclass__ hook -- meaning the metadata is populated when the class is imported, not when an instance is created.

### How __init_subclass__ hooks PYXInspector

# In VectorBlock (base class):

def __init_subclass__(cls, **kwargs):

super().__init_subclass__(**kwargs)

if cls.PYX_FILE is not None:

PYXInspector.populate(cls)   # runs immediately when class body executes

# PYXInspector.populate() scans the .pyx file and fills in:

#   cls.step_func    <- function name containing STEP, COMPUTE, or UPDATE

#   cls.init_func    <- function name containing INIT

#   cls.state_struct <- first ctypedef struct name found

# Only writes if the attribute is currently None (explicit declaration wins)

### What PYXInspector reads from the .pyx file

# Pattern 1: ctypedef struct -> state_struct

# Matches: "ctypedef struct DFC_State_T:"

PATTERN_STRUCT = re.compile(r'ctypedef structs+(w+)')

# Pattern 2: cdef extern function -> step_func or init_func

# Matches: "void DFC_Controller_Step(DFC_State_T *s, ...)"

PATTERN_FUNC   = re.compile(r'(?:void|int|float)s+(w+)s*(')

# Classification:

# INIT in uppercase name    -> init_func

# STEP, COMPUTE, or UPDATE  -> step_func

# RESET                     -> reset_func

# Example .pyx file excerpt:

cdef extern from "embed_sim_dfc_controller.h":

ctypedef struct DFC_State_T:           # -> state_struct = "DFC_State_T"

pass

void DFC_Controller_Init(DFC_State_T *s, float dt_s) nogil   # -> init_func

void DFC_Controller_Step(DFC_State_T *s, ...) nogil           # -> step_func

Explicit declarations always win: if DFControllerBlock explicitly declares step_func = "DFC_Controller_Step" in the class body, PYXInspector will not overwrite it even if it scans the .pyx and finds a different function name. This means that blocks with unusual naming conventions can override the auto-detection without changing the .pyx file.

## 3.4  C_INIT_ARGS -- Typed Literal Injection

C_INIT_ARGS is a list of instance attribute names. At code-generation time, StepGenerator reads the live value of each named attribute from the block instance and emits it as a typed C float literal with the 'f' suffix, satisfying MISRA Rule 7.2.

# Block class:

class SpeedRampBlock(VectorBlock):

C_INIT_ARGS = ["omega_target", "ramp_time"]

# Block instance in simulation file:

speed_ref = SpeedRampBlock("speed_ref",

omega_target = 41.88790205,  # 400 RPM in rad/s

ramp_time    = 0.5)

# Generated Init() call in EmbedSim_step.c:

SpeedRamp_Init(&speed_ref_state, 41.88790205f, 0.50000000f);

#                                ^^^^^^^^^^^^  ^^^^^^^^^^

#                                from speed_ref.omega_target  speed_ref.ramp_time

# For DFControllerBlock with C_INIT_ARGS = ["dt_s"]:

DFC_Controller_Init(&dfc_state, 0.00005000f);

#                               ^^^^^^^^^^^

#                               from dfc.dt_s = 50e-6

Why C_INIT_ARGS works this way:  The block instance exists and has been run during simulation by the time generate_step() is called. All instance attributes have their final, verified values. StepGenerator reads them with getattr(block, attr_name) and formats them as f-suffixed float literals. This means the Init() call always matches the simulation exactly -- there is no separate 'config file' that could drift from the simulation parameters.

Part 4   The Cython Layer: Bridging Python and C

## 4.1  Why Cython

Cython is the bridge layer between Python and C. It serves two distinct purposes in EmbedSim: (1) it provides a high-performance path for blocks that need real-time execution speed during simulation, and (2) it allows the Python simulation to call the exact same C function that will run on the MCU, verifying numerical identity between simulation and production code.

The float[::1] contract is the critical detail. A Cython float[::1] memoryview is a C-contiguous buffer of 32-bit floats. When a Python np.float32 array is passed to a float[::1] parameter, Cython validates the dtype and layout and provides a direct pointer to the buffer's memory -- no copy. When a np.float64 array is passed, Cython does NOT raise an error. It silently succeeds, but the internal buffer remains at zero because the element size (8 bytes) does not match what the C function expects (4 bytes). Every output reads 0.5. The motor hums but does not rotate.

## 4.2  Standard Cython Wrapper Structure

Every EmbedSim Cython wrapper follows a fixed three-method pattern. The pattern is identical across all blocks; only the struct types and function names change.

# cython: language_level=3

# cython: boundscheck=False

# cython: wraparound=False

cimport numpy as cnp

import  numpy as np

# Step 1: declare the C types and functions to Cython

cdef extern from "embed_sim_dfc_controller.h":

ctypedef struct DFC_State_T:

pass                              # Cython only needs the type, not fields

ctypedef struct DFC_Input_T:

float omega_ref_mech

float theta_m

float ia, ib, ic

ctypedef struct DFC_Output_T:

float v_alpha, v_beta

void DFC_Controller_Init(DFC_State_T *s, float dt_s) nogil

void DFC_Controller_Step(DFC_State_T *s,

const DFC_Input_T *u,

float dt,

DFC_Output_T *y) nogil

# Step 2: define the Python-callable wrapper class

cdef class DFCControllerWrapper:

cdef DFC_State_T _state          # C struct allocated on the heap by Cython

cdef float[5]    _u_buf          # fixed-size input buffer

cdef float[2]    _y_buf          # fixed-size output buffer

def __init__(self, float dt_s):

DFC_Controller_Init(&self._state, dt_s)

# set_inputs: receives np.float32 array from Python, zero-copy via float[::1]

cpdef void set_inputs(self, float[::1] u):

# CRITICAL: float[::1] not double[::1]

# If caller passes float64 array, Cython silently zeroes the buffer

self._u_buf[0] = u[0]    # omega_ref_mech

self._u_buf[1] = u[1]    # theta_m

self._u_buf[2] = u[2]    # ia

self._u_buf[3] = u[3]    # ib

self._u_buf[4] = u[4]    # ic

# compute: calls C function with GIL released

cpdef void compute(self, float dt):

cdef DFC_Input_T  u_in

cdef DFC_Output_T y_out

u_in.omega_ref_mech = self._u_buf[0]

u_in.theta_m        = self._u_buf[1]

u_in.ia             = self._u_buf[2]

u_in.ib             = self._u_buf[3]

u_in.ic             = self._u_buf[4]

with nogil:

DFC_Controller_Step(&self._state, &u_in, dt, &y_out)

self._y_buf[0] = y_out.v_alpha

self._y_buf[1] = y_out.v_beta

# get_outputs: returns np.float32 array to Python

cpdef cnp.ndarray get_outputs(self):

cdef cnp.ndarray y = np.empty(2, dtype=np.float32)

y[0] = self._y_buf[0]

y[1] = self._y_buf[1]

return y

cpdef void reset(self):

DFC_Controller_Init(&self._state, 0.00005f)  # re-init with same dt

## 4.3  Build Pipeline

The build pipeline converts the .pyx file into a native .pyd (Windows) or .so (Linux) extension. The scripts build_dfc_controller.bat and build_dfc_controller.sh automate the full sequence:

# What the build script does, in order:

1. Clean stale build artifacts:

del dfc_controller_wrapper*.pyd     <- CRITICAL: stale .pyd takes silent precedence

del dfc_controller_wrapper*.c       <- the Cython-generated C intermediate

rmdir /s /q build                   <- setuptools build directory

2. Run Cython transpiler (via setuptools):

python setup_dfc_controller.py build_ext --inplace

#  .pyx -> .c  (Cython-generated C; 10-50x larger than .pyx)

#  .c   -> .pyd/.so  (C compiler: MSVC on Windows, GCC/Clang on Linux)

3. Copy output to parent directory:

copy dfc_controller_wrapper.cp312-win_amd64.pyd dfc_controller_wrapper.pyd

# The ABI-tagged name changes with Python version; the plain name is stable

# The setup_*.py file:

from setuptools import setup, Extension

from Cython.Build import cythonize

import numpy as np

ext = Extension(

"dfc_controller_wrapper",

sources=["dfc_controller_wrapper.pyx",

"embed_sim_dfc_controller.c"],   # C source compiled in directly

include_dirs=[np.get_include(), "."],

extra_compile_args=["-O2", "-march=native"],

)

Stale build is the most common silent failure:  Setuptools uses file timestamps to decide whether to recompile. If a .pyx file is modified but the .pyd is newer (e.g. because you restored from backup, changed the timestamp, or copied the file), setuptools silently skips the rebuild. The old .pyd loads, presents the old behaviour, and the test passes for the wrong reason. Always delete the .pyd and .c before rebuilding.

## 4.4  Loading the Wrapper at Runtime

The wrapper is loaded lazily in _load_wrapper(), called from __init__ only when use_c_backend=True. The load uses importlib.util.spec_from_file_location() rather than a plain import statement, because the .pyd/.so lives in a directory that is not always on sys.path:

def _load_wrapper(self):

pyd_path = _C_SRC / "dfc_controller_wrapper"  # no extension

try:

# importlib.util handles .pyd on Windows and .so on Linux

spec = importlib.util.spec_from_file_location(

"dfc_controller_wrapper",

str(pyd_path.with_suffix(".pyd" if os.name=="nt" else ".so"))

)

module = importlib.util.module_from_spec(spec)

spec.loader.exec_module(module)

self._wrapper = module.DFCControllerWrapper(

dt_s      = self.dt_s,

# ... all constructor args from instance attributes

)

except (FileNotFoundError, ImportError) as e:

raise RuntimeError(

f"dfc_controller_wrapper not found. Build with:

"

f"  cd {_C_SRC}

"

f"  python setup_dfc_controller.py build_ext --inplace"

) from e

Part 5   StepGenerator: How C is Emitted

## 5.1  What StepGenerator Does

StepGenerator takes a verified, simulated block graph and emits two files: EmbedSim_step.h and EmbedSim_step.c. The generated code is a faithful translation of the Python simulation -- same algorithm, same numerical parameters, same execution order -- but expressed in MISRA C:2012.

StepGenerator is called after sim.run() completes. The block instances are live and have their final attribute values. This is important for C_INIT_ARGS: the emitted Init() literals match the simulation parameters exactly because they are read from the same Python objects that ran the simulation.

## 5.2  DFS Backward Collection

StepGenerator's constructor calls _collect_blocks(), which performs a DFS backward from cg_end to cg_start collecting all blocks in the CodeGen region:

def _collect_blocks(self):

# DFS backward from cg_end, stop at cg_start

# Result: self.blocks = list in topological order (ready for C emission)

visited = set()

order   = []

def dfs(block):

if block in visited:

return

visited.add(block)

for inp in block.inputs:

if inp is self.cg_start:

order.append(inp)   # cg_start is the base case

return

if not getattr(inp, "C_CODEGEN_EXCLUDE", False):

dfs(inp)

order.append(block)

dfs(self.cg_end)

# order is in dependency-resolved sequence: cg_start first, cg_end last

self.blocks = order

## 5.3  Header Generation: EmbedSim_step.h

The header file defines everything the MCU integration layer needs to call EmbedSim_Step(). It contains: the EMBEDSIM_DT macro, all #include directives for block headers, the input struct, the output struct, and the Init and Step prototypes.

### EMBEDSIM_DT macro

# dt_hz is passed as 1.0/DT = 20000.0 for the 20 kHz DFC simulation

# StepGenerator emits:

#define EMBEDSIM_DT (0.00005000f)   /* 1 / 20000.0 Hz = 50 us */

#                    ^^^^^^^^^^^^

#                    Python: f"{1.0/dt_hz:.8f}f"   <- always 8 decimal places

### #include directives -- derived from C_HEADERS

# StepGenerator iterates all blocks in the CodeGen region

# For each block, for each entry in block.C_HEADERS:

#   emit:  #include "<entry.lower()>"

# DFControllerBlock.C_HEADERS = ["embed_sim_dfc_controller.h", "embed_sim_dfc_gains.h"]

# SVPWMBlock.C_HEADERS        = ["svpwm.h"]

# SpeedRampBlock.C_HEADERS    = ["motor_utility_blocks.h"]

# Generated header section:

#include "embed_sim_dfc_controller.h"

#include "embed_sim_dfc_gains.h"

#include "svpwm.h"

#include "motor_utility_blocks.h"

# Note: duplicates are automatically deduplicated by the set() in _collect_headers()

### EmbedSim_Input_T -- assembled from upstream INPUT_NAMES

The input struct is built by _in_sigs(): iterate blocks in reverse order from cg_start, collect any block that has INPUT_NAMES set, and map each name to a struct field using INPUT_KEEP to select which upstream output indices are exposed.

# In db42s02_closed_loop_dfc_20k.py:

# ctrl_packer outputs: [omega_ref_mech, theta_m, ia, ib, ic]

# DFControllerBlock declares:

INPUT_NAMES = ["omega_ref_mech", "theta_m", "ia", "ib", "ic"]

INPUT_KEEP  = [0, 1, 2, 3, 4]      # all 5 signals pass through

C_FIELD_COMMENTS = {

"omega_ref_mech": "Mechanical speed reference [rad/s]",

"theta_m":        "Encoder mechanical angle [rad], accumulating",

"ia":             "Phase-A current [A]",

"ib":             "Phase-B current [A]",

"ic":             "Phase-C current [A]",

}

# Generated struct:

typedef struct {

real32_T omega_ref_mech;  /* Mechanical speed reference [rad/s] */

real32_T theta_m;         /* Encoder mechanical angle [rad], accumulating */

real32_T ia;              /* Phase-A current [A] */

real32_T ib;              /* Phase-B current [A] */

real32_T ic;              /* Phase-C current [A] */

} EmbedSim_Input_T;

#              ^^^^^^^^^^^

#              field name comes directly from INPUT_NAMES[i]

#              always real32_T unless C_INPUT_TYPES overrides

### EmbedSim_Output_T -- assembled from downstream OUTPUT_NAMES

# SVPWMBlock declares:

OUTPUT_NAMES   = ["ta", "tb", "tc", "sector"]

OUTPUT_KEEP    = [0, 1, 2, 3]

C_OUTPUT_TYPES = {"sector": "uint8_T"}   # overrides default real32_T for this field

# Generated struct:

typedef struct {

real32_T ta;       /* Phase A duty cycle [0, 1] */

real32_T tb;       /* Phase B duty cycle [0, 1] */

real32_T tc;       /* Phase C duty cycle [0, 1] */

uint8_T  sector;   /* Active SVPWM sector 0..5  */

#   ^^^^^^^

#   StepGenerator reads C_OUTPUT_TYPES.get("sector", "real32_T")

#   and uses it instead of the default

} EmbedSim_Output_T;

## 5.4  Source Generation: EmbedSim_step.c

The source file contains: static state declarations, EmbedSim_Init(), and EmbedSim_Step(). The Init and Step bodies are built by iterating self.blocks in topological order and calling _emit_block() for each.

### Static state declarations -- block.state_struct + block.name

# StepGenerator iterates blocks and emits:

# for block in self.blocks:

#     if block.state_struct is not None:

#         emit: "static {block.state_struct} {block.name}_state;"

# Generated declarations:

static DFC_State_T      dfc_state;

static SpeedRamp_T      speed_ref_state;

static SVPWM_State_T    svpwm_state;

#      ^^^^^^^^^^^^      ^^^^^^^^^^^^

#      state_struct      block.name + "_state"

# "static" enforces MISRA Rule 8.7: internal linkage for file-scope objects

### EmbedSim_Init() -- typed literal arguments from C_INIT_ARGS

# StepGenerator.generate_init():

# for block in self.blocks:

#     if block.init_func is not None:

#         args = [f"{getattr(block, a):.8f}f" for a in block.C_INIT_ARGS]

#         emit: "{block.init_func}(&{block.name}_state, {', '.join(args)});"

void EmbedSim_Init(void) {

DFC_Controller_Init(&dfc_state, 0.00005000f);

#                                   ^^^^^^^^^^^

#                                   from dfc.dt_s = 50e-6

SpeedRamp_Init(&speed_ref_state, 41.88790205f, 0.30000000f);

#                                    ^^^^^^^^^^^^  ^^^^^^^^^^

#                                    omega_target  ramp_time

#                                    (live instance attributes read at codegen time)

}

### _emit_block() -- the decision tree

For each block, _emit_block() applies a decision tree in priority order. The first matching condition determines what is emitted:

| Priority | Condition | What is emitted | Example |
| --- | --- | --- | --- |
| 1 | C_CUSTOM_EMIT is set | Verbatim C string from C_CUSTOM_EMIT attribute | DFControllerBlock: DFC_Controller_Step() + normalisation |
| 2 | IS_UNIT_DELAY is True | memcpy from previous-step y[] buffer (z^-1 register) | VectorDelay blocks inside CodeGen region |
| 3 | NUM_INPUTS == 0 | Source block pattern: Step(&state, dt, y) with no u[] array | SpeedRampBlock, VectorConstant |
| 4 | step_func is set | Normal pattern: build u[] then call Step(&state, u, dt, y) | SVPWMBlock, coordinate transforms |
| 5 | C_SOURCES set, no step_func | TODO comment in generated code | Incomplete block: needs step_func |
| 6 | None of the above | Python-only comment: block has no C implementation | Diagnostic / logging blocks |

### Normal block emission -- building u[] from upstream y[] buffers

# StepGenerator._emit_block() for a normal block (priority 4):

# 1. Declare y[] output buffer

real32_T y_svpwm_pack[3];   /* y_<block.name>[<OUTPUT_SIZE>] */

# 2. Build u[] input buffer from upstream blocks

real32_T u_svpwm_pack[2];   /* u_<block.name>[sum of upstream OUTPUT_SIZEs] */

u_svpwm_pack[0] = y_dfc[0]; /* v_alpha from DFC block */

u_svpwm_pack[1] = y_dfc[1]; /* v_beta  from DFC block */

#                 ^^^^^^^^^

#                 upstream.name + "[" + index + "]"

#                 index from upstream.OUTPUT_KEEP if set, else sequential

# 3. Call the step function

SVPWMPack_Step(&svpwm_pack_state, u_svpwm_pack, EMBEDSIM_DT, y_svpwm_pack);

#              ^^^^^^^^^^^^^^^^^^  ^^^^^^^^^^^^                ^^^^^^^^^^^^

#              block.name+"_state" "u_"+block.name             "y_"+block.name

### C_CUSTOM_EMIT -- when and why it exists

The C_CUSTOM_EMIT attribute is an escape hatch for blocks whose C function has a non-standard ABI -- one that StepGenerator cannot automatically handle with the standard Step(&state, u, dt, y) pattern. DFControllerBlock is the canonical example: its C function outputs physical voltages [V], but SVPWMPackBlock expects normalised references in [-1,+1]. The normalisation y_dfc[0] = y_dfc_out.v_alpha / DFC_V_MAX cannot be expressed as a generic Step() call.

# DFControllerBlock.C_CUSTOM_EMIT:

C_CUSTOM_EMIT = """

/* --- dfc_controller (DFControllerBlock) --- */

/* DFC_Controller_Step() outputs physical voltages [V].                */

/* The SVPWM block expects normalised [-1,+1] references.              */

/* Divide by DFC_V_MAX = DFC_V_DC / sqrt(3) before SVPWM.             */

DFC_Input_T   u_dfc;

DFC_Output_T  y_dfc_out;

real32_T      y_dfc[2];

u_dfc.omega_ref_mech = in->omega_ref_mech;   /* <- reads from Input_T */

u_dfc.theta_m        = in->theta_m;

u_dfc.ia             = in->ia;

u_dfc.ib             = in->ib;

u_dfc.ic             = in->ic;

DFC_Controller_Step(&dfc_state, &u_dfc, dt, &y_dfc_out);

/* Normalise: physical [V] -> SVPWM [-1, +1] */

y_dfc[0] = y_dfc_out.v_alpha / DFC_V_MAX;

y_dfc[1] = y_dfc_out.v_beta  / DFC_V_MAX;

"""

# Note: C_CUSTOM_EMIT is responsible for declaring y_dfc[2]

# so the next block (SVPWMPackBlock) can reference y_dfc[0] and y_dfc[1]

Rule: C_CUSTOM_EMIT is not a workaround, it is a contract:  When you use C_CUSTOM_EMIT, you take full responsibility for: declaring the y_<name>[] output buffer so downstream blocks can read it, reading from in-> for Input_T fields, writing correct units (the downstream block expects a specific unit convention), and maintaining MISRA C:2012 compliance. If StepGenerator's standard path could handle your block, it should. Every unnecessary C_CUSTOM_EMIT is a maintenance liability.

### Pack section -- writing EmbedSim_Output_T

After all blocks have been emitted, StepGenerator emits the pack section: for each OUTPUT_NAMES entry on the final output block, read the corresponding index from the output block's y_ buffer and write it to the output struct field, applying the C type cast specified by C_OUTPUT_TYPES:

/* Pack section -- written by _emit_pack() */

out->ta     =          y_svpwm[0];   /* real32_T: default */

out->tb     =          y_svpwm[1];   /* real32_T: default */

out->tc     =          y_svpwm[2];   /* real32_T: default */

out->sector = (uint8_T)y_svpwm[3];   /* uint8_T: from C_OUTPUT_TYPES["sector"] */

#             ^^^^^^^

#             StepGenerator emits cast only when C_OUTPUT_TYPES overrides

#             the default real32_T for that field

### Unpack section -- reading EmbedSim_Input_T

The first thing EmbedSim_Step() does is read from in-> and place the values into the y[] buffers of the first blocks in the CodeGen region. These blocks then appear to have outputs already computed when downstream blocks are emitted:

void EmbedSim_Step(float dt, const EmbedSim_Input_T *in, EmbedSim_Output_T *out) {

/* Unpack section -- built by _emit_unpack() */

/* ctrl_packer block provides the Input_T signals */

real32_T y_ctrl_packer[5];

y_ctrl_packer[0] = in->omega_ref_mech;  /* INPUT_KEEP[0] = 0 */

y_ctrl_packer[1] = in->theta_m;         /* INPUT_KEEP[1] = 1 */

y_ctrl_packer[2] = in->ia;              /* INPUT_KEEP[2] = 2 */

y_ctrl_packer[3] = in->ib;              /* INPUT_KEEP[3] = 3 */

y_ctrl_packer[4] = in->ic;              /* INPUT_KEEP[4] = 4 */

/* Now downstream blocks can reference y_ctrl_packer[i] normally */

/* Block emissions follow in topological order ... */

Part 6   FMU and Modelica: Physics as Code

## 6.1  Why FMU is a First-Class Citizen

The Modelica/FMU layer is not a bolted-on extension. It occupies the same architectural role in EmbedSim that the C layer occupies: a technology with a precise, non-overlapping responsibility. The C layer executes the controller. The FMU layer executes the plant. These two things are symmetric: both are compiled artifacts, both are driven by automatically-generated Python wrappers, and both are invisible to the simulation wiring code.

The symmetry is important: the engineer who writes the Modelica model never sees FMPy. The engineer who writes the controller never sees the FMU binary. Each writes in their natural language (Modelica equations, Python block logic) and the framework handles the glue.

## 6.2  The Complete FMU Pipeline

Step 1: Write PMSM_Plant_FMU.mo  (Modelica source)

Physical PMSM model: dq-frame voltage equations + torque + mechanics

Standard FMI 2.0 connectors: input/output/parameter declarations

Step 2: Compile to PMSM_Plant_FMU.fmu  (OpenModelica: omc)

.fmu is a ZIP archive containing:

modelDescription.xml   <- interface definition (inputs, outputs, params)

binaries/linux64/*.so  <- compiled Modelica C code

sources/PMSM_Plant_FMU.c  <- generated C (readable, not hand-written)

resources/*.bin        <- Jacobian matrices for DASSL solver

Step 3: Run gen_fmu.py  (one-shot, run whenever .mo changes)

Reads modelDescription.xml from .fmu (or .mo via MoParser)

Calls ClientGenerator to emit PMSM_Plant_FMUBlock.py

-> VectorBlock subclass with INPUT_VARS, OUTPUT_VARS, DEFAULT_PARAMS

Step 4: Simulation uses PMSM_Plant_FMUBlock as the plant

FMPy wraps the .so binary: set_real(inputs), do_step(t, dt), get_real(outputs)

EmbedSim calls this once per simulation step

C_CODEGEN_EXCLUDE = True: never enters StepGenerator

## 6.3  modelDescription.xml -- The FMU Interface Contract

The modelDescription.xml inside the .fmu ZIP is the FMI 2.0 standard interface contract. It is the only file that gen_fmu.py needs to read -- the binary .so can be replaced or recompiled without changing the Python wrapper as long as the interface contract does not change.

| XML element | What it contains | Mapped to |
| --- | --- | --- |
| modelName | PMSM_Plant_FMU | VectorBlock class name: PMSM_Plant_FMUBlock |
| ScalarVariable causality=input | duty_a, duty_b, duty_c, v_dc, T_load | FMUBlock.INPUT_VARS list |
| ScalarVariable causality=output | rpm, ia, ib, ic, theta_m, T_em, id_out, iq_out | FMUBlock.OUTPUT_VARS list |
| ScalarVariable causality=parameter | R, L_d, L_q, lambda_pm, J, B_fric, p, v_dc_nom | FMUBlock.DEFAULT_PARAMS dict |
| Real start= attribute | 0.19, 1.25e-4, 0.0014 ... | DEFAULT_PARAMS values |

## 6.4  gen_fmu.py -- How the Wrapper is Generated

gen_fmu.py uses two internal classes. MoParser reads the .mo Modelica source as plain text (no Modelica compiler needed) and extracts the interface. ClientGenerator renders the VectorBlock subclass using Python string templates.

# gen_fmu.py in full:

parser   = MoParser(mo_text)         # parse PMSM_Plant_FMU.mo

model    = parser.parse()             # model.name, model.inputs, model.outputs, model.parameters

generator     = ClientGenerator(model)

generated_code = generator.generate()

# The generated class (PMSM_Plant_FMUBlock.py):

class PMSM_Plant_FMUBlock(FMUBlock):

INPUT_VARS    = ['duty_a', 'duty_b', 'duty_c', 'v_dc', 'T_load']

OUTPUT_VARS   = ['rpm', 'ia', 'ib', 'ic', 'theta_m', 'T_em', 'id_out', 'iq_out']

DEFAULT_PARAMS = {

'R':         0.19,

'L_d':       0.000125,

'L_q':       0.000125,

'lambda_pm': 0.0014,

'J':         2.4e-6,

'B_fric':    1e-6,

'p':         4,

'v_dc_nom':  17.0

}

def __init__(self, name, fmu_path):

super().__init__(name=name, fmu_path=fmu_path,

input_names=self.INPUT_VARS,

output_names=self.OUTPUT_VARS,

parameters=self.DEFAULT_PARAMS,

instance_name=name)

## 6.5  FMUBlock Base Class -- The FMPy Integration

FMUBlock is the EmbedSim base class for all FMU-based plant blocks. It wraps FMPy's FMU2Slave API into the standard compute_py() interface that the simulation engine calls. The FMU runs in co-simulation mode (FMI 2.0 CS): EmbedSim provides inputs, triggers a time step, reads outputs.

class FMUBlock(VectorBlock):

C_CODEGEN_EXCLUDE = True   # FMU always stays outside CodeGen boundary

def compute_py(self, t, dt, input_values):

# 1. Unpack VectorSignal -> flat float list

values = [float(v) for v in input_values[0].value]

# 2. Set FMU inputs via FMPy

self._fmu.setReal(self._input_vrefs, values)

#                 ^^^^^^^^^^^^^^^^^^

#                 value references from modelDescription.xml

#                 looked up by name during __init__

# 3. Advance FMU by one timestep

self._fmu.doStep(t, dt)

# 4. Read FMU outputs

outputs = self._fmu.getReal(self._output_vrefs)

# 5. Wrap in VectorSignal

self.output = VectorSignal(np.array(outputs, dtype=DEFAULT_DTYPE), self.name)

return self.output

## 6.6  PMSM_Python_Plant -- Portable FMU Alternative

When the FMU binary is not available (different OS, ARM target, unit testing), PMSM_Python_Plant provides an identical interface using a pure-Python RK4 integration of the dq-frame PMSM equations. The class has C_CODEGEN_EXCLUDE = True and the same 8-element output bus [rpm, ia, ib, ic, theta_m, T_em, id, iq]. Controller code runs against either plant without modification.

Key design principle: all coordinate transforms inside PMSM_Python_Plant are delegated to coordinate_transform_blocks.py -- the same Python mirrors of the canonical C functions. There is no inline transform math anywhere in the plant. This means the Python plant uses the same transform conventions as the controller, preventing sign or scaling mismatches.

# PMSM_Python_Plant uses transform blocks, not inline math:

self._ct_clarke     = ClarkeTransformBlock("_plant_clarke",    use_c_backend=False)

self._ct_park       = ParkTransformBlock("_plant_park",        use_c_backend=False)

self._ct_inv_park   = InvParkTransformBlock("_plant_inv_park", use_c_backend=False)

self._ct_inv_clarke = InvClarkeTransformBlock("_plant_inv_clarke", use_c_backend=False)

# In _vdq_from_duties():

ab  = self._ct_clarke.compute_py(0.0, 0.0, [inp])

dq  = self._ct_park.compute_py(0.0, 0.0, [ab_sig, th_sig])

# Same Clarke/Park code path as the controller

# Transform bugs manifest identically in both plant and controller -> easy to spot

Part 7   Three Examples: From Engine to Full Production

Example 1   example_algebraic_loop.py   --   Engine Fundamentals

The purpose of this example is to make the engine's internal mechanics visible. Read the Part 3 docstring in the example file alongside this section.

### What the Example Shows You

Three things that are invisible in FOC examples because they are handled automatically:

- DFS loop detection: the engine rejects an invalid diagram at construction time, before any computation. You see the exact ValueError and understand why it fires.

- VectorDelay mechanics: you see the exact execution order (delay first, before its upstream gain), understand why this is correct, and observe the zero-fallback warning at t=0.

- Heun integration: the recurrence y(k) = K * [A*sin(...) + y(k-1)] is a first-order IIR. You can verify the stability condition |K| < 1 by trying K=0.5 (stable) and K=1.1 (diverges). The ODESolver choice does not affect stability for this system, but affects accuracy for nonlinear systems.

### The Diagnostic Pattern

Part 3 of the example prints a step-by-step trace of what the engine does at t=0 and t=0.01. This is the most direct way to understand _compute_all_blocks(). The trace shows exactly which block runs, what its inputs are, and what its output becomes. When you encounter unexpected simulation behaviour in FOC examples, adding this kind of trace (via SimulationLogger) is the first debugging step.

Example 2   db42s02_openloop_fmu.py   --   CodeGen Boundary + FMU

The purpose of this example is to make the CodeGen boundary visible and to show the FMU plant in a complete simulation loop.

### The Boundary in Practice

In this example, the boundary is placed after SVPWMPackBlock. This is intentional: the AURIX ISR can compute sqrtf() and atan2f() faster than EmbedSim can represent them as VectorBlocks. The simulation verifies the full signal chain including the Cartesian-to-polar conversion; only the SVPWM sector calculation is pushed to the MCU.

Moving the boundary is a one-line wiring change. To move it earlier (absorb InvPark into the MCU):

# Original Rev 3.0 boundary:

inv_park >> svpwm_pack >> cg_start >> svpwm >> cg_end

# Future Rev 4.0 boundary (InvPark inside CodeGen):

inv_park >> cg_start >> svpwm_pack >> svpwm >> cg_end

# EmbedSim_Input_T now needs: { v_alpha, v_beta } instead of { magnitude, angle_rad }

# Only the wiring changes. InvPark block implementation does not change.

### What DB42S02PlantBlock Teaches About C_CODEGEN_EXCLUDE

The DB42S02PlantBlock subclass is the cleanest example of the exclude mechanism. Notice that it:

- sets C_CODEGEN_EXCLUDE = True -- it is a simulation-only object

- receives the same [ta, tb, tc, sector] bus that the AURIX ISR will produce

- injects V_DC = 17.0 and T_LOAD = 0.01 as physical constants -- these are never CodeGen signals

- re-packs a stable 5-element scope bus [rpm, ia, ib, ic, T_em] -- independent of FMU's 8-element output order

This pattern teaches you how to write any simulation-only block: make it C_CODEGEN_EXCLUDE, give it the same input interface as the real hardware, inject physical constants internally, and expose a stable, well-documented scope bus.

### Reading the Topology HTML

The example calls sim.topo.export_html() to produce an interactive block diagram. The HTML file shows the CodeGen boundary visually: blocks inside the boundary are highlighted, blocks outside are greyed. Wire labels (set via _wire_labels dict in the example) annotate every signal with its physical meaning and units. This HTML is the single best tool for verifying that the block diagram matches your intent before running the simulation.

Example 3   db42s02_closed_loop_dfc_20k.py   --   Full Production Stack

This is the complete production EmbedSim application. Every mechanism described in Parts 1-6 appears here in final form. Read it as a verification that the architecture works.

### The Feedback Loop as VectorDelay at Scale

In Example 1, VectorDelay broke a 2-block feedback loop. Here it breaks an 8-signal bus feedback loop spanning the entire controller chain. The principle is exactly the same; only the scale changes.

# The feedback topology:

motor >> motor_delay >> ctrl_packer >> cg_start >> dfc >> svpwm_pack >> svpwm >> cg_end >> motor

#               ^                                                                          ^

#               |                                                                          |

#               +-----(z^-1)-------[8 signals: rpm, ia, ib, ic, theta_m, Tem, id, iq]----+

#

# DFS execution order (motor_delay is a LoopBreaker):

#   motor_delay, ctrl_packer, cg_start, dfc, svpwm_pack, svpwm, cg_end, motor

#

# At step k:

#   motor_delay outputs motor(k-1)  <- stored from previous step

#   ctrl_packer uses motor(k-1) to build the DFC input bus

#   dfc computes v_alpha, v_beta from motor(k-1) state

#   svpwm computes duties from v_alpha, v_beta

#   motor advances using duties(k) -> produces motor(k)

#   motor_delay latches motor(k) for step k+1

### The Noise Model as a Plant Decorator

The AURIX noise model is implemented entirely inside DB42S02PlantBlock.compute_py() as a post-processing step on the plant's outputs. This is the correct architectural place: noise is a hardware property of the sensor chain, not a property of the controller. The controller receives the noisy signals and must handle them; the simulation verifies that it does.

The noise model calls helper functions (_adc_noise, _pwm_spike, _enc_quantise, _enc_glitch, _deadtime_disturbance, _vdc_ripple) each of which models one specific hardware non-ideality. Each is independently testable by setting with_noise=False in _run_sim() -- the clean plant runs the same controller without any noise, providing a baseline for performance comparison.

### The NN Tuner as a Simulation Consumer

The NN surrogate tuner is the best example of EmbedSim being used as a platform, not just a tool. The tuner calls _run_sim() 40+ times in Phase 1 (LHS exploration), each call running the complete 5-second closed-loop simulation. It consumes simulation results as training data for the MLP surrogate, then optimises the surrogate to find better gains, then calls _run_sim() one more time to verify the recommended gains.

The key architectural point: the tuner is not special. It is ordinary Python code that calls the same _run_sim() function as the manual test. The simulation is a pure Python function that takes parameters and returns a result dict. This composability -- simulation as function -- is what makes automated gain optimisation, Monte Carlo analysis, and hardware-in-the-loop testing all natural extensions of the same simulation code.

### CodeGen in the DFC Example -- What is Different from Example 2

In Example 2, the CodeGen region contained only SVPWMBlock. In this example it contains the full DFC controller. The generated EmbedSim_Step() now includes:

- Unpack: y_ctrl_packer[] from in->omega_ref_mech ... in->ic

- C_CUSTOM_EMIT: DFC_Controller_Step() called with DFC_Input_T / DFC_Output_T structs + voltage normalisation

- Normal: SVPWMPack_Step() called with y_dfc[] as input

- Normal: SVM_CalculateDutyCycle() called with y_svpwm_pack[] as input

- Pack: out->ta / out->tb / out->tc / out->sector from y_svpwm[]

The input struct changed from { magnitude, angle_rad } (Rev 3.0 open-loop) to { omega_ref_mech, theta_m, ia, ib, ic } (DFC closed-loop). This boundary change required: one different CodeGenStart position in the wiring; INPUT_NAMES / INPUT_KEEP set on DFControllerBlock; nothing else.

Part 8   Reference

## 8.1  Complete VectorBlock Metadata Reference

| Attribute | Type | Set by | Read by | Description |
| --- | --- | --- | --- | --- |
| PYX_FILE | str | Block author | PYXInspector at import | Path to .pyx; triggers auto-population |
| step_func | str | PYXInspector or explicit | StepGenerator | C step function name |
| state_struct | str | PYXInspector or explicit | StepGenerator | C state struct type |
| init_func | str | PYXInspector or explicit | StepGenerator | C init function name |
| C_SOURCES | list | Block author | StepGenerator (compilation) | .c files compiled into firmware |
| C_HEADERS | list | Block author | StepGenerator (emit #include) | Headers, auto-lowercased |
| NUM_INPUTS | int | Block author | StepGenerator | 0 = source block, N = normal |
| OUTPUT_SIZE | int | Block author | StepGenerator | Length of y[] output buffer |
| C_INIT_ARGS | list | Block author | StepGenerator | Instance attr names -> Init() literals |
| C_CUSTOM_EMIT | str | Block author | StepGenerator | Verbatim C for non-standard ABI |
| C_CODEGEN_EXCLUDE | bool | Block author | StepGenerator | True = never emitted |
| INPUT_NAMES | list | Block author or instance | StepGenerator | Input_T field names |
| INPUT_KEEP | list | Block author or instance | StepGenerator | Upstream y[] indices for Input_T |
| C_INPUT_TYPES | dict | Block author | StepGenerator | C type per Input_T field |
| OUTPUT_NAMES | list | Block author or instance | StepGenerator | Output_T field names |
| OUTPUT_KEEP | list | Block author or instance | StepGenerator | y[] indices for Output_T |
| C_OUTPUT_TYPES | dict | Block author | StepGenerator | C type per Output_T field |
| C_FIELD_COMMENTS | dict | Block author | StepGenerator | /* comments */ for Input_T fields |
| TOPO_CATEGORY | str | Block author | TopologyPrinter | Visual category in HTML diagram |

## 8.2  Generated C Name Derivation Summary

| Generated C name | Derivation rule | Concrete example |
| --- | --- | --- |
| State variable | block.name + "_state" | dfc -> dfc_state |
| Output buffer | "y_" + block.name | y_dfc |
| Input buffer | "u_" + block.name | u_svpwm_pack |
| Init call | init_func("&"+name+"_state", *C_INIT_ARGS_values) | DFC_Controller_Init(&dfc_state, 0.00005f) |
| Step call | step_func("&"+name+"_state", u, dt, y) | DFC_Controller_Step(&dfc_state, ...) |
| Input_T field | INPUT_NAMES[i] | omega_ref_mech, theta_m, ia |
| Output_T field | OUTPUT_NAMES[i] | ta, tb, tc, sector |
| Output_T type | C_OUTPUT_TYPES.get(name, "real32_T") | sector -> uint8_T |
| #include path | C_HEADERS[i].lower() | embed_sim_dfc_controller.h |
| Timestep macro | 1.0/dt_hz formatted as .8f + "f" | 0.00005000f |
| MISRA f-suffix | Applied to all Init() float literals | 41.88790205f not 41.88790205 |

## 8.3  Glossary

| Term | Definition |
| --- | --- |
| ATOM | Advanced Timer Output Module -- GTM sub-module, 24-bit, CMU clock, ARU-connected. |
| C_CODEGEN_EXCLUDE | Block attribute. True -> block never enters StepGenerator. |
| C_CUSTOM_EMIT | Verbatim C fragment for non-standard ABI. Responsible for declaring y_<name>[] buffer. |
| C_INIT_ARGS | List of instance attr names read at codegen time and emitted as typed f-suffixed float literals. |
| ClientGenerator | gen_fmu.py class: renders VectorBlock subclass Python code from a parsed Modelica model. |
| CodeGen boundary | The region between CodeGenStart and CodeGenEnd translated to C by StepGenerator. |
| DFC | Differential Flatness Controller -- model-based feedforward law cancelling plant dynamics. |
| DEFAULT_DTYPE | numpy.float32 -- all EmbedSim signal arrays use this dtype. |
| FMI 2.0 | Functional Mock-up Interface 2.0 -- the standard API for FMU co-simulation. |
| FMPy | Python library wrapping FMI 2.0 .so/.dll binaries via ctypes. |
| FMU | Functional Mock-up Unit -- Modelica model compiled to a self-contained ZIP binary. |
| float[::1] | Cython C-contiguous float32 memoryview. Mandatory in all set_inputs(). Silently zeroes on float64 input. |
| gen_fmu.py | One-shot utility: reads .mo or modelDescription.xml, emits VectorBlock FMU wrapper. |
| LoopBreaker | VectorBlock subclass marker. DFS cuts back-edges at LoopBreaker instances. |
| MoParser | gen_fmu.py class: parses Modelica .mo source as text to extract inputs/outputs/parameters. |
| MISRA C:2012 | Coding standard. Key rules: 7.2 (f suffix on floats), 8.7 (static linkage), 15.5 (single return). |
| PYXInspector | Regex-based .pyx parser. Runs at class-definition time via __init_subclass__. |
| real32_T | typedef float real32_T -- single-precision IEEE 754. Used throughout generated C. |
| SMO | Sliding Mode Observer -- back-EMF estimation in alphabeta frame for sensorless speed. |
| SpeedFusion | Adaptive blend of encoder speed and SMO speed, weight = f(¦omega¦). |
| StepGenerator | Emits EmbedSim_step.c/.h from the CodeGenStart/End region after DFS collection. |
| SVPWM | Space Vector PWM -- alphabeta voltage -> three-phase duty cycles. |
| VectorDelay | z^-1 LoopBreaker. Outputs y(k-1) at step k. The universal algebraic loop solution. |
| VectorSignal | Thin wrapper around np.float32 array. The type of every block.output. |