EmbedSim — Fachschale Tutorial

Buck Converter: Modelica → FMU → C → Cython → Python Block

Complete five-layer workflow — physics equations to Aurix TC3xx ISR

EmbedSim Framework  ·  March 2026

# 1.  What Is a Fachschale?

A Fachschale (domain layer) is EmbedSim's unit of reusable engineering IP. It is a self-contained Python package encapsulating a domain so it can be plugged into any simulation graph, validated against a high-fidelity FMU, and its C code generated for Aurix TC3xx.

| Layer | Purpose / File |
| --- | --- |
| 0 — Modelica model | BuckConverter.mo — physics as equations; compiled to .fmu by OpenModelica |
| 1 — FMU block | BuckConverterBlock.py — auto-generated by mo_to_fmu_client.py; wraps .fmu via FMI 2.0 |
| 2 — C source | buck_converter.c / .h — MISRA-C plant; identical binary on PC and Aurix |
| 3 — Cython wrapper | buck_wrapper.pyx — zero-copy bridge to the C functions |
| 4 — Python block | buck_block.py — VectorBlock subclass; compute() override; wires into the graph |

💡  Layers 0–1 are for high-fidelity simulation. Layers 2–4 are the production path — the same C binary runs on PC (via Cython) and on Aurix (compiled by TASKING/HighTec).

# 2.  Real Project Layout

Your project already has the reference Fachschale in electrical_blocks\. The Buck Converter Fachschale follows the same layout, creating a new package at project root:

C:\EmbedSimProject\

├── electrical_blocks\              REFERENCE Fachschale (existing)

│   ├── speed_pi_block.py           Layer 4 — overrides compute(t, dt, input_values)

│   ├── smc_block.py

│   ├── modelica\

│   │   └── mo_to_fmu_client.py     FMU block generator (master copy)

│   └── c_src\

│       ├── speed_pi_controller.c / .h

│       ├── speed_pi_wrapper.pyx

│       └── setup_speed_pi.py

│

├── buck_converter\                 NEW Fachschale (this tutorial)

│   ├── __init__.py

│   ├── buck_block.py               Layer 4 — overrides compute(t, dt, input_values)

│   └── c_src\

│       ├── Sys_Types.h             copy from electrical_blocks\c_src\

│       ├── buck_converter.h        Layer 2

│       ├── buck_converter.c        Layer 2

│       ├── buck_wrapper.pyx        Layer 3

│       └── setup_buck.py

│

└── examples\buck_converter\       Example scripts (already exists)

├── BuckConverterBlock.py       Layer 1 — auto-generated, already present

├── c_src\                     currently empty — .pyd deployed here after build

└── modelica\

├── BuckConverter.mo        Layer 0 — Modelica source

├── BuckConverter.fmu       already compiled

└── mo_to_fmu_client.py

# 3.  The compute() Pattern — How EmbedSim Blocks Work

This is the most important section to understand before writing buck_block.py. Every hand-written block in your project — OpenLoopStartupBlock, MotorOutputBlock, NoisySensorBlock, etc. — uses the same pattern:

## 3.1  What to override: compute() directly

Hand-written blocks override compute(self, t, dt, input_values). This is the method EmbedSim calls each simulation step. The block reads from input_values[port].value, does its work, sets self.output = VectorSignal(array, self.name), and returns it:

# Pattern used by ALL hand-written blocks in your project

class MyBlock(VectorBlock):

def __init__(self, name):

super().__init__(name)

self.vector_size = 2          # declare output size

def compute(self, t, dt, input_values=None):

# Read from upstream block (port 0)

u = input_values[0].value if input_values else np.zeros(2)

# Your logic here

y = u * 2.0

# Must always set self.output and return it

self.output = VectorSignal(np.array(y, dtype=np.float32), self.name)

return self.output

def reset(self):                  # optional — reset state for next run

super().reset()

self._state = 0.0

## 3.2  What NOT to use for hand-written blocks

The compute_py(), compute_c(), and SimBlockBase split is only for auto-generated CodeGen stubs emitted by generate_pyx_stub(). They add complexity without benefit for hand-written blocks where you control both paths. Use the clean pattern above.

| Method | When to use |
| --- | --- |
| compute(t, dt, input_values) | Hand-written blocks — you always override this |
| compute_py() / compute_c() | Auto-generated stubs only — CodeGen output, not input |
| SimBlockBase | Only used in pmsm_foc_pwm_smc.py imports for the CodeGen markers; not for your block class |

💡  BuckBlock uses compute() not compute_py(). The C backend (BuckWrapper) is selected inside compute() by checking self._wrap is not None — exactly as NoisySensorBlock checks its RNG, or MotorOutputBlock checks the input length.

# 4.  Layer 0 — Modelica Model  (BuckConverter.mo)

The file at examples\buck_converter\modelica\BuckConverter.mo is the authoritative physics source. Every C implementation must reproduce these equations exactly:

model BuckConverter

"Simple average-value buck converter — CCM"

parameter Real L      = 100e-6  "Inductance [H]";

parameter Real C      = 100e-6  "Capacitance [F]";

parameter Real R_load = 10      "Load resistance [Ohm]";

parameter Real V_in   = 24      "Input voltage [V]";

input  Real duty      "PWM duty cycle [0-1]";

output Real V_out     "Output capacitor voltage [V]";

output Real I_L       "Inductor current [A]";

output Real I_load    "Load current [A]";

protected

Real switch_state;

equation

switch_state = duty;

L * der(I_L)   = switch_state * V_in - V_out;

C * der(V_out) = I_L - V_out / R_load;

I_load = V_out / R_load;   // algebraic — no der()

initial equation

I_L   = 0;

V_out = 0;

end BuckConverter;

| Equation | Physical meaning |
| --- | --- |
| L · der(I_L) = duty·V_in − V_out | Inductor dynamics — volt-second balance |
| C · der(V_out) = I_L − V_out/R | Capacitor dynamics — current balance |
| I_load = V_out / R_load | Algebraic — computed directly, no integration |

# 5.  Layer 1 — FMU Block  (BuckConverterBlock.py)

BuckConverterBlock.py already exists in examples\buck_converter\. It was generated by mo_to_fmu_client.py and wraps the DASSL-integrated BuckConverter.fmu. Regenerate it after changing the .mo:

> cd examples\buck_converter\modelica

> python mo_to_fmu_client.py BuckConverter.mo

The FMU block has four output variables: V_out, I_L, I_load, switch_state. Only the first three are used in validation against BuckBlock.

⚠  The FMU block uses the OpenModelica DASSL variable-step solver. It runs only on Win/Linux x64. The C layer below provides the Aurix-deployable equivalent.

# 6.  Layer 2 — C Source  (buck_converter\c_src\)

## 6.1  buck_converter.h

Copy Sys_Types.h from electrical_blocks\c_src\ first. Then create the header — variable names match the Modelica model exactly:

/* buck_converter.h — MISRA-C:2012 | ASIL-B */

#ifndef BUCK_CONVERTER_H

#define BUCK_CONVERTER_H

#include "Sys_Types.h"   /* real32_T = float */

/* State: I_L (inductor) + V_out (capacitor) */

typedef struct {

real32_T iL;    /* I_L   [A] */

real32_T vC;    /* V_out [V] */

} BuckState_T;

/* Input per step */

typedef struct {

real32_T duty;  /* PWM duty cycle  [0..1] */

real32_T Vin;   /* V_in  [V]              */

real32_T R;     /* R_load [Ohm]           */

} BuckInput_T;

void     Buck_Init  (BuckState_T *s);

void     Buck_Step  (BuckState_T *s, const BuckInput_T *u,

real32_T dt, real32_T L, real32_T C);

real32_T Buck_Vout  (const BuckState_T *s);

real32_T Buck_IL    (const BuckState_T *s);

real32_T Buck_Iload (const BuckState_T *s, real32_T R);

#endif /* BUCK_CONVERTER_H */

## 6.2  buck_converter.c

The _deriv() function is a direct one-to-one translation of the two Modelica der() lines. Compare each C line with the .mo equation section — they must be identical in structure:

/* buck_converter.c */

#include "buck_converter.h"

/* _deriv — translates Modelica equation section:

*   L * der(I_L)   = duty * V_in - V_out  →  *diL = (duty*Vin - vC) / L

*   C * der(V_out) = I_L - V_out / R_load  →  *dvC = (iL - vC/R) / C   */

static void _deriv(const BuckState_T *s, const BuckInput_T *u,

real32_T L, real32_T C,

real32_T *diL, real32_T *dvC)

{

*diL = (u->duty * u->Vin - s->vC) / L;

*dvC = (s->iL  - s->vC / u->R)   / C;

}

/* Buck_Init — Modelica: initial equation  I_L = 0; V_out = 0 */

void Buck_Init(BuckState_T *s) { s->iL = 0.0f; s->vC = 0.0f; }

/* Buck_Step — RK4 fixed step */

void Buck_Step(BuckState_T *s, const BuckInput_T *u,

real32_T dt, real32_T L, real32_T C)

{

real32_T k1_iL, k1_vC, k2_iL, k2_vC, k3_iL, k3_vC, k4_iL, k4_vC;

BuckState_T tmp;

_deriv(s,    u, L, C, &k1_iL, &k1_vC);

tmp.iL = s->iL + 0.5f*dt*k1_iL; tmp.vC = s->vC + 0.5f*dt*k1_vC;

_deriv(&tmp, u, L, C, &k2_iL, &k2_vC);

tmp.iL = s->iL + 0.5f*dt*k2_iL; tmp.vC = s->vC + 0.5f*dt*k2_vC;

_deriv(&tmp, u, L, C, &k3_iL, &k3_vC);

tmp.iL = s->iL +     dt*k3_iL; tmp.vC = s->vC +     dt*k3_vC;

_deriv(&tmp, u, L, C, &k4_iL, &k4_vC);

s->iL += (dt/6.0f)*(k1_iL + 2.0f*k2_iL + 2.0f*k3_iL + k4_iL);

s->vC += (dt/6.0f)*(k1_vC + 2.0f*k2_vC + 2.0f*k3_vC + k4_vC);

}

real32_T Buck_Vout (const BuckState_T *s)             { return s->vC; }

real32_T Buck_IL   (const BuckState_T *s)             { return s->iL; }

real32_T Buck_Iload(const BuckState_T *s, real32_T R) { return s->vC / R; }

⚠  Pass L and C as parameters — not globals. Two independent converter instances must never share a BuckState_T. This mirrors the pattern in speed_pi_controller.c.

# 7.  Layer 3 — Cython Wrapper  (buck_wrapper.pyx)

Mirrors speed_pi_wrapper.pyx. The private cdef BuckState_T _state is allocated on the Python object heap — no GIL overhead per step() call:

# buck_wrapper.pyx — cython: language_level=3, boundscheck=False

cdef extern from "buck_converter.h":

ctypedef struct BuckState_T:

float iL

float vC

ctypedef struct BuckInput_T:

float duty; float Vin; float R

void  Buck_Init  (BuckState_T *s)

void  Buck_Step  (BuckState_T *s, const BuckInput_T *u, float dt, float L, float C)

float Buck_Vout  (const BuckState_T *s)

float Buck_IL    (const BuckState_T *s)

float Buck_Iload (const BuckState_T *s, float R)

cdef class BuckWrapper:

cdef BuckState_T _state   # private C struct — zero Python overhead

cdef float _L

cdef float _C

def __cinit__(self, float L=1e-4, float C=1e-4):

self._L = L; self._C = C

Buck_Init(&self._state)

def reset(self): Buck_Init(&self._state)

def step(self, float duty, float Vin, float R, float dt):

"""Returns (V_out, I_L, I_load) — matches BuckConverter.mo outputs."""

cdef BuckInput_T u

u.duty = duty; u.Vin = Vin; u.R = R

Buck_Step(&self._state, &u, dt, self._L, self._C)

return (Buck_Vout(&self._state),

Buck_IL  (&self._state),

Buck_Iload(&self._state, R))

@property

def V_out(self): return Buck_Vout(&self._state)

@property

def I_L(self):   return Buck_IL  (&self._state)

def set_L(self, float v): self._L = v   # follows set_integrator() pattern

def set_C(self, float v): self._C = v

## Build and deploy

> cd buck_converter\c_src

> python setup_buck.py build_ext --inplace

> copy buck_wrapper.cp312-win_amd64.pyd ..\

> copy buck_wrapper.cp312-win_amd64.pyd ..\..\examples\buck_converter\c_src\

# 8.  Layer 4 — Python Block  (buck_block.py)

## 8.1  Architecture decision: compute() not compute_py()

BuckBlock overrides compute(self, t, dt, input_values). This is the same method signature used by every hand-written block in your project. Inside compute(), the C backend is selected by checking self._wrap is not None:

| Condition | Path taken |
| --- | --- |
| self._wrap is not None | True: call self._wrap.step(duty, Vin, R, dt) — BuckWrapper (Cython) |
| self._wrap is None | False: call self._py_rk4(duty, Vin, R) — pure Python RK4 |

The wrapper is created in reset(). If the .pyd is not built, self._wrap stays None and the Python path runs silently. This matches the graceful fallback in speed_pi_block.py and svpwm_block.py.

## 8.2  Complete buck_block.py

# buck_converter/buck_block.py

from __future__ import annotations

import sys, numpy as np

from pathlib import Path

_HERE = Path(__file__).resolve().parent

_ROOT = _HERE.parent

for _p in [str(_ROOT), str(_HERE)]:

if _p not in sys.path: sys.path.insert(0, _p)

from embedsim.core_blocks import VectorBlock, VectorSignal

sys.path.insert(0, str(_HERE / "c_src"))

try:

from buck_wrapper import BuckWrapper as _BuckWrapper

_C_AVAILABLE = True

except ImportError:

_BuckWrapper = None; _C_AVAILABLE = False

class BuckBlock(VectorBlock):

"""

VectorBlock wrapping the average-value Buck Converter.

Physics from: examples/buck_converter/modelica/BuckConverter.mo

Inputs  (port 0, size 3):  [duty, Vin, R_load]

Outputs          (size 3):  [V_out, I_L, I_load]

compute() selects backend inside the same method — matching the pattern

of all hand-written blocks in the project (NoisySensorBlock, etc.).

NOT SimBlockBase — that is for auto-generated CodeGen stubs only.

"""

C_INCLUDE   = '"buck_converter.h"'

C_SIGNATURE = ("void Buck_Step(BuckState_T *s, const BuckInput_T *u,"

" real32_T dt, real32_T L, real32_T C);")

C_INPUT_MAP = [("cg_start", 0), ("cg_start", 1), ("cg_start", 2)]

def __init__(self, name, L=100e-6, C=100e-6, R_load=10.0,

V_in=24.0, use_c_backend=True, **kwargs):

super().__init__(name, **kwargs)

self.vector_size  = 3

self.output_label = "[V_out, I_L, I_load]"

self._L = float(L); self._C = float(C)

self._R_default   = float(R_load)

self._Vin_default = float(V_in)

self._dt          = None

self._wrap        = None        # set in reset() if .pyd available

self._iL          = 0.0        # Python fallback state

self._vC          = 0.0

self._use_c       = use_c_backend and _C_AVAILABLE

def reset(self, dt=None):

super().reset()

if dt is not None: self._dt = float(dt)

if self._use_c:

self._wrap = _BuckWrapper(L=self._L, C=self._C)  # C path

else:

self._iL = 0.0; self._vC = 0.0                  # Python path

def compute(self, t, dt, input_values=None):

if self._dt is None: self._dt = float(dt)

if input_values and input_values[0] is not None:

u    = input_values[0].value

duty = float(np.clip(u[0], 0.0, 1.0))

Vin  = float(u[1]) if len(u) > 1 else self._Vin_default

R    = float(u[2]) if len(u) > 2 else self._R_default

else:

duty = 0.0; Vin = self._Vin_default; R = self._R_default

# ── C backend ────────────────────────────────────────────────

if self._wrap is not None:

V_out, I_L, I_load = self._wrap.step(duty, Vin, R, self._dt)

# ── Python fallback ──────────────────────────────────────────

else:

V_out, I_L = self._py_rk4(duty, Vin, R)

I_load = V_out / R

self.output = VectorSignal(

np.array([V_out, I_L, I_load], dtype=np.float32), self.name)

return self.output

def _py_rk4(self, duty, Vin, R):

"""Pure-Python RK4 — numerically identical to Buck_Step in C."""

dt, L, C = self._dt, self._L, self._C

def _d(iL, vC):

return (duty*Vin - vC)/L, (iL - vC/R)/C

k1 = _d(self._iL,                  self._vC)

k2 = _d(self._iL+.5*dt*k1[0],     self._vC+.5*dt*k1[1])

k3 = _d(self._iL+.5*dt*k2[0],     self._vC+.5*dt*k2[1])

k4 = _d(self._iL+   dt*k3[0],     self._vC+   dt*k3[1])

self._iL += (dt/6)*(k1[0]+2*k2[0]+2*k3[0]+k4[0])

self._vC += (dt/6)*(k1[1]+2*k2[1]+2*k3[1]+k4[1])

return self._vC, self._iL   # V_out, I_L

def set_R_load(self, R): self._R_default = float(R)

def set_V_in (self, V): self._Vin_default = float(V)

# 9.  Validation — FMU vs C

Run both backends on identical inputs and diff the outputs. This confirms the C equations are a faithful translation of the Modelica physics. Place the script in examples\buck_converter\:

# validate_c_vs_fmu.py — key section

def run_sim(plant, label):

inp  = BuckInputBlock()   # outputs [duty=0.5, Vin=24.0, R=10.0]

sink = VectorEnd("sink")

inp >> plant >> sink

sim = EmbedSim(sinks=[sink], T=5e-3, dt=1e-5, solver=ODESolver.RK4)

sim.scope.add(plant, label=label)

sim.run()

# collect columns: V_out, I_L, I_load

return np.column_stack([

sim.scope.data.get(f"{label}[{i}]", []) for i in range(3)])

c_out   = run_sim(BuckBlock("c", use_c_backend=True), "c")

fmu_out = run_sim(BuckConverterBlock("fmu",

fmu_path="modelica/BuckConverter.fmu"), "fmu")

diff = np.abs(c_out - fmu_out[:len(c_out), :3])

print(f"Max V_out error: {diff[:,0].max()*1000:.3f} mV")   # accept < 1 mV

print(f"Max I_L  error: {diff[:,1].max()*1000:.3f} mA")    # accept < 1 mA

| Metric | Accept if | Notes |
| --- | --- | --- |
| V_out error | < 1 mV | RK4 fixed-step vs DASSL variable-step solver tolerance |
| I_L error | < 1 mA | Same cause — integrator tolerance difference, not equation error |
| > 10 mV | FAIL | Check signs in _deriv() line-by-line against Modelica der() equations |

# 10.  Code Generation — Aurix TC3xx

Add CodeGenStart / CodeGenEnd markers around the chain exactly as in pmsm_foc_pwm_smc.py. C_INPUT_MAP maps the three input registers to duty, Vin, R_load:

# In your buck simulation script — follows pmsm_foc_pwm_smc.py pattern

from embedsim.code_generator import CodeGenStart, CodeGenEnd

cg_start = CodeGenStart("cg_start")

cg_end   = CodeGenEnd  ("cg_end")

plant.C_INPUT_MAP = [

("cg_start", 0),   # duty

("cg_start", 1),   # Vin

("cg_start", 2),   # R_load

]

cg_start >> plant >> cg_end

result = cg_end.generate_loop(

cg_start   = cg_start,

output_dir = _PROJECT_ROOT,   # writes into embedsim_gen/

dt_hz      = 1.0 / DT,

write_files= True,

)

## Aurix ISR integration

/* isr_buck.c — Aurix TC3xx 100 kHz ISR */

#include "embedsim_loop.h"

void ISR_100kHz(void)

{

motor_reg[0] = g_duty_setpoint;            /* duty from supervisor */

motor_reg[1] = DSADC_GetResult(1) * SCALE; /* Vin  from ADC       */

motor_reg[2] = g_R_load_estimate;          /* R    from param table*/

embedsim_loop_step(EMBEDSIM_DT);           /* runs Buck_Step       */

GTM_ATOM_SetDuty(0, buck_out[0]);          /* V_out monitor        */

}

# 11.  Completion Checklist

- BuckConverter.mo — equations present, initial conditions iL=0, vC=0

- BuckConverterBlock.py — generated from .mo, already in examples\buck_converter\

- Sys_Types.h — copied from electrical_blocks\c_src\

- buck_converter.h — BuckState_T, BuckInput_T, Buck_Step prototype

- buck_converter.c — _deriv() matches Modelica equation section line-by-line

- buck_wrapper.pyx — cdef class BuckWrapper, step() returns (V_out, I_L, I_load)

- setup_buck.py — builds cleanly, .pyd produced

- .pyd copied to buck_converter\ and examples\buck_converter\c_src\

- buck_block.py — compute() overrides, C backend selected by self._wrap is not None

- validate_c_vs_fmu.py — V_out < 1 mV, I_L < 1 mA

- generate_loop() — embedsim_loop.c with no TODO comments

# 12.  Troubleshooting

| Symptom | Fix |
| --- | --- |
| V_out error > 10 mV vs FMU | Check sign in _deriv() — compare with Modelica der() equations one-to-one |
| ImportError: buck_wrapper | Rebuild setup_buck.py --inplace, copy .pyd to both target paths |
| V_out = 0 every step | Block not in sim.add() — reset() never called |
| self._wrap is None at runtime | _C_AVAILABLE is False — .pyd not found on sys.path in reset() |
| Algebraic loop error | Add VectorDelay on the V_out feedback path to a PI controller |
| TODO in embedsim_loop.c | C_INPUT_MAP indices wrong — check register assignments |

EmbedSim Framework  ·  Buck Converter Fachschale  ·  March 2026