EmbedSim Framework

Fachschale Tutorial

Buck Converter — PI Voltage Controller

A complete guide: Modelica plant → C algorithm → Cython bridge → Python block → CodeGen → Plots

EmbedSim v3.1.0  |  March 2026

# 1.  What Is a Fachschale?

The German term Fachschale ("domain shell") describes a self-contained module that bundles all layers needed for one embedded control function. In EmbedSim a Fachschale always consists of four layers:

| Layer | Purpose |
| --- | --- |
| Plant model (.mo / .fmu) | Physics — Modelica ODE compiled to FMI 2.0 FMU. Simulates the system under control. |
| C controller (.c / .h) | Algorithm — portable MISRA-C, zero OS dependency. Identical binary on Aurix TriCore and ARM Cortex-M4. |
| Cython wrapper (.pyx) | Bridge — exposes the C struct to Python as a cdef class. Float32, near-zero overhead, no GIL. |
| Python block (.py) | Orchestration — derives from SimBlockBase. Plugs into VectorSim, drives RK4, carries CodeGen metadata. |

The critical benefit: the C controller is never rewritten. The same source file runs in simulation via Cython and is compiled with TASKING ctc and flashed to the Aurix ECU.

# 2.  System Overview

## 2.1  Closed-Loop Signal Flow

Signal flow — PI voltage regulation

V_ref ────────────> +----------+  duty [0..1]  +------------------+

|          |────────────>  |                  |

V_meas (fdbk) ─────>| PI_Buck  |               | BuckConverter    |

|  Block   |               | FMU (plant)      |

+----------+               |                  |

^                     | V_out  I_L  I_load|

|   VectorDelay       +------------------+

+<──── V_out (delayed 1 step) ──────────+

The one-step VectorDelay on the feedback path is mandatory. It breaks the algebraic loop so EmbedSim's DFS topological scheduler can assign a strict execution order without iteration.

## 2.2  File Map

| File | Location | Role |
| --- | --- | --- |
| BuckConverter.mo | buck_converter/modelica/ | Modelica plant ODE |
| BuckConverter.fmu | buck_converter/modelica/ | Compiled FMI 2.0 unit |
| BuckConverterBlock.py | buck_converter/ | Auto-generated FMUBlock class |
| pi_buck_controller.c/h | electrical_blocks/c_src/ | Portable MISRA-C PI algorithm |
| pi_buck_wrapper.pyx | electrical_blocks/c_src/ | Cython bridge (float32) |
| setup_pi_buck.py | electrical_blocks/c_src/ | Cython build script |
| build_pi_buck.bat | electrical_blocks/c_src/ | Compile and copy .pyd |
| pi_buck_block.py | buck_converter/ | VectorBlock (RK4 + CodeGen) |
| pi_buck_example.py | examples/pi_buck_converter/ | Runnable simulation demo |

# 3.  Plant Model (BuckConverter.mo)

The plant is the physical system under control. In EmbedSim it is always modelled in OpenModelica and compiled to an FMU. The auto-generator mo_to_fmu_client.py then writes a typed FMUBlock Python class from the parsed .mo source.

## 3.1  Modelica Equations

BuckConverter.mo

model BuckConverter

"Simple buck converter plant model"

parameter Real L       = 1e-4  "Inductance [H]";

parameter Real C       = 1e-4  "Capacitance [F]";

parameter Real R_load  = 10.0  "Load resistance [Ohm]";

parameter Real V_in    = 24.0  "Input voltage [V]";

parameter Real f_sw    = 1e5   "Switching frequency [Hz]";

input  Real duty   "PWM duty cycle [0-1]";

output Real V_out  "Output voltage [V]";

output Real I_L    "Inductor current [A]";

output Real I_load "Load current [A]";

Real V_sw;

equation

V_sw  = duty * V_in;

L * der(I_L)    = V_sw - V_out;   // inductor ODE

C * der(V_out)  = I_L - I_load;   // capacitor ODE

I_load = V_out / R_load;           // resistive load

end BuckConverter;

Steady-state operating point: V_out = duty × V_in. For V_out = 12 V, V_in = 24 V the duty converges to 0.5.

## 3.2  Generating the FMU Block (gen_fmu.py)

gen_fmu.py

from mo_to_fmu_client import generate_fmu_block

mo_path    = r'C:\EmbedSimProject\buck_converter\modelica\BuckConverter.mo'

output_dir = r'C:\EmbedSimProject\buck_converter'

output_file = generate_fmu_block(mo_path, output_dir, verbose=True)

# Writes: buck_converter/BuckConverterBlock.py

| NOTE | Re-run gen_fmu.py whenever BuckConverter.mo changes. The generated file carries a DO NOT EDIT header — always regenerate rather than edit manually. |
| --- | --- |

# 4.  C Controller (pi_buck_controller.c)

The PI algorithm is written in portable C with no platform dependencies. It targets Infineon Aurix TriCore as primary and ARM Cortex-M4 as secondary. The same source file runs unchanged in simulation via the Cython wrapper.

## 4.1  Algorithm

PI with anti-windup — discrete recurrence

e(k)      = V_ref(k) - V_meas(k)

integ(k)  = clamp( integ(k-1) + e(k)*dt,

-duty_max/Ki, +duty_max/Ki )  // anti-windup

duty(k)   = clamp( Kp*e(k) + Ki*integ(k),

duty_min, duty_max )

The integrator clamp is the anti-windup mechanism: it prevents Ki×integ from exceeding duty_max regardless of how long the error persists.

## 4.2  Data Structures

pi_buck_controller.h — structs

typedef struct {

real32_T Kp;        // Proportional gain [1/V]

real32_T Ki;        // Integral gain [1/(V*s)]

real32_T duty_max;  // Max duty cycle [0-1]

real32_T duty_min;  // Min duty cycle [0-1]

real32_T Ts;        // Nominal sample time [s]

} PI_Buck_Params_T;

typedef struct {

real32_T integrator;   // Accumulated error integral

real32_T prev_error;   // Previous error (diagnostics)

real32_T last_output;  // Last duty cycle

} PI_Buck_State_T;

typedef struct {           // One instance per channel

PI_Buck_Params_T params;

PI_Buck_State_T  state;

} PI_Buck_Block_T;

typedef struct { real32_T V_ref;  real32_T V_meas; } PI_Buck_Input_T;

typedef struct { real32_T duty; }                   PI_Buck_Output_T;

| WHY real32_T? | real32_T (= float) comes from Sys_Types.h, the EmbedSim standard header adopted from Simulink Coder ERT for Aurix TriCore. Using it throughout guarantees bit-identical float32 arithmetic between simulation and ECU firmware. |
| --- | --- |

## 4.3  API

pi_buck_controller.h — function prototypes

void PI_Buck_Init      (PI_Buck_Block_T* pPI);

void PI_Buck_SetParams (PI_Buck_Block_T* pPI,

real32_T Kp, real32_T Ki,

real32_T duty_max, real32_T duty_min,

real32_T Ts);

void PI_Buck_ResetState(PI_Buck_Block_T* pPI);

void PI_Buck_Compute   (PI_Buck_Block_T* pPI,

const PI_Buck_Input_T* pIn,

real32_T dt,

PI_Buck_Output_T* pOut);

# 5.  Cython Wrapper (pi_buck_wrapper.pyx)

The Cython wrapper bridges C and Python. PI_Buck_Block_T is stored by value inside the cdef class — no heap allocation, no pointer crossing the Python/C boundary.

## 5.1  Wrapper Class

pi_buck_wrapper.pyx

# distutils: sources = pi_buck_controller.c

# cython: language_level = 3

cdef class PI_BuckWrapper:

cdef PI_Buck_Block_T _block   # stored by value

def __cinit__(self): PI_Buck_Init(&self._block)

def set_params(self, float Kp, float Ki,

float duty_max, float duty_min, float Ts):

PI_Buck_SetParams(&self._block, Kp, Ki, duty_max, duty_min, Ts)

def compute(self, float V_ref, float V_meas, float dt) -> float:

cdef PI_Buck_Input_T  inp

cdef PI_Buck_Output_T out

inp.V_ref  = V_ref

inp.V_meas = V_meas

PI_Buck_Compute(&self._block, &inp, dt, &out)

return out.duty

# RK4 state synchronisation

def set_integrator(self, float v): self._block.state.integrator = v

def get_integrator(self) -> float: return self._block.state.integrator

def get_last_output(self) -> float: return self._block.state.last_output

def reset(self): PI_Buck_ResetState(&self._block)

## 5.2  Compiling

Build commands

cd C:\EmbedSimProject\electrical_blocks\c_src

REM Option A: compile + auto-copy .pyd

build_pi_buck.bat

REM Option B: manual

python setup_pi_buck.py build_ext --inplace

| COMPILE ONCE | After a successful build the .pyd is a self-contained binary. Recompile only when pi_buck_controller.c or pi_buck_wrapper.pyx changes. |
| --- | --- |

# 6.  Python Block (pi_buck_block.py)

PI_BuckBlock derives from SimBlockBase and handles three concerns: RK4 state integration, dual Python/C backend dispatch, and CodeGen metadata for embedsim_loop.c emission.

## 6.1  CodeGen Attributes

PYXInspector reads these class-level attributes when generating embedsim_loop.c:

pi_buck_block.py — CodeGen metadata

class PI_BuckBlock(SimBlockBase):

PYX_FILE     = '.../c_src/pi_buck_wrapper.pyx'  # absolute path

step_func    = 'PI_Buck_Compute'                 # C function name

state_struct = 'PI_Buck_Block_T'                 # C state struct

NUM_INPUTS   = 2                                 # V_ref, V_meas

OUTPUT_SIZE  = 1                                 # duty

C_SOURCES    = ['pi_buck_controller.c']

C_HEADERS    = ['pi_buck_controller.h', 'Sys_Types.h']

## 6.2  RK4 State Integration

get_derivative() — RK4 interface

self.state = np.zeros(1, dtype=np.float32)  # state[0] = integrator

def get_derivative(self, t, input_values=None):

"""dx/dt = error — RK4 integrates this to produce integ(t)"""

if t < self._t_enable:          # soft-start guard

return np.zeros(1, dtype=np.float32)

V_ref, V_meas = self._get_voltages(input_values)

return np.array([V_ref - V_meas], dtype=np.float32)

## 6.3  Dual Backend Dispatch

Backend selection and C↔Python state sync

# Python backend — no compilation needed

pi = PI_BuckBlock('pi', Kp=0.15, Ki=8.0, use_c_backend=False)

# C backend — requires compiled pi_buck_wrapper.pyd

pi = PI_BuckBlock('pi', Kp=0.15, Ki=8.0, use_c_backend=True)

def compute_c(self, t, dt, input_values):

self._impl.set_integrator(np.float32(self.state[0]))  # Python→C

duty = self._impl.compute(V_ref, V_meas, dt)

self.state[0] = np.float32(self._impl.get_integrator())  # C→Python

# 7.  Code Generator (code_generator.py)

The EmbedSim code generator emits embedsim_loop.c and embedsim_loop.h into embedsim_gen/ at project root. These files are the MCU deployment artefact — a self-contained C loop compilable with TASKING ctc for Aurix TriCore.

## 7.1  Architecture

| Class | File | Responsibility |
| --- | --- | --- |
| SimBlockBase | embedsim/code_generator.py | Base class; carries PYX_FILE, step_func, C_SOURCES, C_HEADERS |
| PYXInspector | electrical_blocks/pyx_inspector.py | Parses .pyx; extracts C function signatures and port counts |
| CodeGenStart | embedsim/code_generator.py | Transparent passthrough; marks INPUT boundary of generated region. Wired inline with >>. |
| CodeGenEnd | embedsim/code_generator.py | Transparent passthrough; marks OUTPUT boundary of generated region. Wired inline with >>. |
| LoopGenerator | embedsim/code_generator.py | Walks sorted block list between CodeGenStart and CodeGenEnd; calls _emit_block() for each SimBlockBase; writes embedsim_loop.c / .h |
| _emit_block() | embedsim/code_generator.py | Emits one C function call with explicit signal wiring |
| _gen_h() | embedsim/code_generator.py | Emits #include directives and extern register declarations |

## 7.2  Triggering Code Generation

CodeGenStart and CodeGenEnd are transparent VectorBlock passthroughs. They must be wired inline into the signal graph with the >> operator, exactly like any other block. After sim.run() completes, call LoopGenerator.generate() to emit the files.

Correct CodeGen pattern — inline wiring + LoopGenerator

from embedsim.code_generator import CodeGenStart, CodeGenEnd, LoopGenerator

# ── Step 1: create boundary markers ────────────────────────────────

cg_start = CodeGenStart('pi_ctrl_start')

cg_end   = CodeGenEnd('pi_ctrl_end')

# ── Step 2: wire inline into the graph ──────────────────────────

v_ref >> cg_start >> pi_controller >> cg_end >> buck_plant >> sink

# ── Step 3: run simulation (boundary markers are pass-through) ──

sim.run(verbose=True)

# ── Step 4: emit C files ───────────────────────────────────────

gen = LoopGenerator(cg_start, cg_end)

gen.generate(output_dir='C:\\EmbedSimProject\\embedsim_gen', dt_hz=1e6)

# Writes:

#   embedsim_gen/embedsim_loop.c

#   embedsim_gen/embedsim_loop.h

| IMPORTANT | CodeGenStart and CodeGenEnd are NOT context managers. Do not use: with CodeGenStart(): sim.run() — this will fail. They must be wired with >> before sim is created, and LoopGenerator called after sim.run() completes. |
| --- | --- |

## 7.3  Generated C Loop

embedsim_loop.c (representative output)

/* embedsim_loop.c

* Auto-generated by EmbedSim LoopGenerator (feature 05121967)

* DO NOT EDIT — re-generate with LoopGenerator.generate()

*/

#include <string.h>   /* memcpy */

#include "embedsim_loop.h"

#include "pi_buck_controller.h"

void embedsim_loop_step(real32_T dt)

{

/* Block: pi_buck (PI_BuckBlock) */

real32_T u_pi_buck[2];

u_pi_buck[0] = y_vref[0];

u_pi_buck[1] = y_fb_delay[0];

real32_T y_pi_buck[1];

PI_Buck_Compute(&pi_buck_state, u_pi_buck, dt, y_pi_buck);

/* FMU blocks omitted on MCU target */

}

| MCU NOTE | FMU blocks (OpenModelica plant models) are simulation-only and are omitted from embedsim_loop.c. Only SimBlockBase-derived blocks with C_SOURCES / C_HEADERS are emitted. The LoopGenerator DFS traversal visits only blocks inside the cg_start → cg_end boundary. |
| --- | --- |

## 7.4  Generated Header

embedsim_loop.h

/* embedsim_loop.h

* Auto-generated by EmbedSim LoopGenerator (feature 05121967)

*/

#ifndef EMBEDSIM_LOOP_H

#define EMBEDSIM_LOOP_H

#include "Sys_Types.h"   /* real32_T */

/* Sample period for a 1000000 Hz control loop */

#define EMBEDSIM_DT  (0.0000010000f)

#include "pi_buck_controller.h"

/* Persistent state (file-scope statics — one per stateful block) */

static PI_Buck_Block_T pi_buck_state;

void embedsim_loop_init(void);

void embedsim_loop_step(real32_T dt);

#endif /* EMBEDSIM_LOOP_H */

## 7.5  Common CodeGen Pitfalls

| Error / Symptom | Root Cause | Fix |
| --- | --- | --- |
| ImportError: cannot import name 'CodeGenRegion' | CodeGenRegion was an early prototype name, removed in v3. | Import CodeGenStart, CodeGenEnd, LoopGenerator instead. |
| 3 block(s) in region instead of 1 | cg_start and cg_end passthrough blocks are counted by the DFS traversal. | Expected behaviour — passthrough blocks count. Only PI_BuckBlock emits a real C call. |
| embedsim_loop.c contains only /* [pass-through] */ comments | Block is missing PYX_FILE / step_func class attributes, or does not derive from SimBlockBase. | Ensure the block class declares step_func, state_struct, C_SOURCES, C_HEADERS. |
| UnicodeEncodeError on Windows when writing .c / .h | write_text() defaulted to cp1252 instead of UTF-8. | Fixed in code_generator.py v1.6: all write_text() calls use encoding='utf-8'. |
| AttributeError: 'VectorScope' has no attribute 'time' | VectorScope stores time in .t not .time. | Use scope.t (list) or scope._buf_t[:scope._step_idx] (pre-allocated path). |
| scope.data keys not found | Multi-index signals are stored as 'label[0]', 'label[1]', not plain 'label'. | Look up f"{label}[{index}]" first, fall back to plain label for scalars. |

# 8.  Running the Simulation (pi_buck_example.py)

pi_buck_example.py assembles all blocks into a closed-loop VectorSim graph, prints the topology, runs the simulation, triggers CodeGen, and plots the results.

## 8.1  Imports and Block Construction

§1 — Block construction

from embedsim.simulation_engine import EmbedSim, ODESolver, VectorDelay

from embedsim.source_blocks     import VectorStep

from embedsim.dynamic_blocks    import VectorEnd

from embedsim.code_generator    import CodeGenStart, CodeGenEnd, LoopGenerator

from embedsim.topology_printer  import TopologyPrinter

v_ref         = VectorStep('vref', step_time=0.001,

before_value=0.0, after_value=12.0, dim=1)

pi_controller = PI_BuckBlock(name='pi_buck', Kp=0.15, Ki=8.0,

duty_max=0.9, duty_min=0.1, Ts=1e-4,

use_c_backend=True)

buck_plant    = BuckConverterBlock(name='buck', fmu_path=...,

L=100e-6, C=100e-6, R_load=10,

V_in=24, f_sw=100e3)

feedback_delay = VectorDelay('fb_delay', initial=[0.0])

sink           = VectorEnd('sink')

cg_start       = CodeGenStart('pi_ctrl_start')

cg_end         = CodeGenEnd('pi_ctrl_end')

## 8.2  Wiring

§2 — Wiring

# Forward path with CodeGen boundary markers inline

v_ref         >> cg_start          # V_ref enters codegen region

cg_start      >> pi_controller     # V_ref → PI port 0

pi_controller >> cg_end            # duty exits codegen region

cg_end        >> buck_plant        # duty → FMU input

buck_plant    >> sink

# Feedback path

buck_plant     >> feedback_delay   # V_out → delay

feedback_delay >> pi_controller    # delayed V_meas → PI port 1

## 8.3  Topology Printer

§3 — Topology printer

sim = EmbedSim(sinks=[sink], T=0.01, dt=1e-6, solver=ODESolver.RK4)

printer = TopologyPrinter(sim, title='Buck Converter — PI Voltage Control')

printer.print_console()   # ASCII diagram to stdout

printer.show_gui()        # GUI window (close to continue)

| NOTE | printer.show_gui() opens a GUI block-diagram window before the run starts. Close the window to allow execution to continue to sim.run(). |
| --- | --- |

## 8.4  Simulation and CodeGen

§4 — Simulation + CodeGen

sim.scope.add(v_ref,          label='v_ref')

sim.scope.add(pi_controller,  label='pi_ctrl')

sim.scope.add(buck_plant,     label='buck_out', indices=[0, 1])

sim.scope.add(feedback_delay, label='fb_delay')

sim.run(verbose=True, progress_bar=True)

# Emit C files after the run

gen = LoopGenerator(cg_start, cg_end)

gen.generate(output_dir=codegen_dir, dt_hz=1e6)

# → embedsim_gen/embedsim_loop.c

# → embedsim_gen/embedsim_loop.h

## 8.5  Scope Data Access (PlotHelper)

VectorScope stores recorded data in two possible forms depending on whether the pre-allocation path was used. PlotHelper handles both transparently:

Scope data access pattern

scope = sim.scope

# Time — list path (small sims) or pre-allocated ndarray (large sims)

t_raw = scope.t if scope.t else scope._buf_t[:scope._step_idx]

t_ms  = np.asarray(t_raw) * 1e3   # convert to milliseconds

# Data keys — multi-index signals stored as 'label[0]', 'label[1]'

key = f"{label}[{index}]"         # e.g. 'buck_out[0]' for V_out

if key not in scope.data and label in scope.data:

y = np.array([float(v) for v in scope.data[label]])  # scalar

else:

y = np.array([float(v) for v in scope.data[key]])    # indexed

## 8.6  Simulation Results

Response to a 12 V step at t = 1 ms. Parameters: Kp = 0.15, Ki = 8.0, L = 100 µH, C = 100 µF, R_load = 10 Ω, V_in = 24 V.

Observations:

- V_out rises from 0 V and tracks toward 12 V. Overshoot (~16 V) results from the aggressive Ki = 8.0 setting.

- Duty cycle saturates at 90 % (duty_max) during the transient, then settles near 45 % — consistent with the ideal 12/24 = 0.5 operating point.

- Residual oscillation at 10 ms indicates the gains need further reduction. See Section 9.

- Inductor current I_L mirrors V_out and stabilises near 1.2 A (= 12 V / 10 Ω).

# 9.  Tuning Guide

The five parameters below are the primary tuning knobs. Changes take effect via set_params() — no recompilation needed.

## 9.1  Parameter Reference

| Parameter | Default | Effect | Adjust if... |
| --- | --- | --- | --- |
| Kp | 0.15 | Proportional gain: immediate duty correction per volt of error. | Overshoot too large → decrease. Sluggish rise → increase. |
| Ki | 8.0 | Integral gain: eliminates steady-state offset over time. | Residual offset → increase. Growing oscillation → decrease. |
| duty_max | 0.90 | Hard ceiling on duty cycle. Protects inductor from saturation. | Inductor saturates at startup → reduce to 0.75–0.80. |
| duty_min | 0.10 | Hard floor. Avoids discontinuous conduction mode at light load. | Output collapses at light load → try 0.05. |
| Ts | 1e-4 | Nominal sample time. Used when dt = 0 is passed to Compute(). | Mismatch with simulation dt causes integration drift. |

## 9.2  Systematic Tuning Sequence

- Set Ki = 0. Increase Kp until V_out rises fast with acceptable overshoot (< 15 %).

- Freeze Kp. Increase Ki from 0 until steady-state offset disappears within 3–5 ms.

- If oscillation grows, halve Ki. If offset remains, double Ki.

- Tighten duty_max to the largest duty needed at steady state plus 10 % headroom.

- Verify both backends (use_c_backend=True and False) give the same steady-state voltage.

## 9.3  Runtime Update

Runtime tuning — no restart needed

pi_controller.set_params(Kp=0.08, Ki=4.0)  # reduced gains

sim.reset()

sim.run()

pi_controller.set_params(duty_max=0.80)     # tighter ceiling

# 10.  Creating a New Fachschale

The Buck Converter pattern generalises to any controller. Follow this checklist:

- Write the Modelica plant (.mo) with explicit input/output/parameter declarations. Compile to FMU. Run gen_fmu.py.

- Write the controller in plain C using real32_T. Structure it as Init / SetParams / Reset / Compute with anti-windup on all integrator paths.

- Write the Cython .pyx wrapper as a cdef class storing the C struct by value. Add set_integrator / get_integrator for every continuous state.

- Write the Python block deriving from SimBlockBase. Populate PYX_FILE, step_func, state_struct, C_SOURCES and C_HEADERS. Implement get_derivative() for each continuous state.

- Add setup_xxx.py and build_xxx.bat following the pi_buck pattern. Run build_xxx.bat.

- Wire a simulation graph and verify steady-state against the analytical operating point.

- Wire CodeGenStart and CodeGenEnd around the target blocks (Section 7.2) and call LoopGenerator.generate(). Verify embedsim_loop.c compiles clean with TASKING ctc.

# 11.  Quick-Reference Card

End-to-end command sequence from Modelica source to MCU-ready C output:

Full workflow — project root

# 1. Generate FMUBlock from Modelica source

python gen_fmu.py

# 2. Compile Cython extension

cd electrical_blocks\c_src

build_pi_buck.bat

cd ..\..\

# 3. Run simulation + CodeGen (all-in-one)

cd examples\pi_buck_converter

python pi_buck_example.py

# Output: embedsim_gen/embedsim_loop.c

#         embedsim_gen/embedsim_loop.h

# 4. Build all extensions from project root

cd C:\EmbedSimProject && build_all.bat

| ★ NEXT | The PMSM FOC example in examples/pmsm/ applies the same pattern to a full FOC chain: Clarke/Park, SVPWM, SMC current controller, SMO observer, and Speed PI — all wired with >> under one RK4 engine. |
| --- | --- |