EmbedSim

Framework for Embedded Control Engineers

A Beginner's Guide

Introduction · Motivation · Conceptual Architecture

https://github.com/vectorsim/embed_sim_project

# 1  What is EmbedSim?

EmbedSim is an open-source Python framework that lets you design, simulate, and generate production C code for embedded control systems — all from a single, readable Python script. Think of it as a lightweight, programmer-friendly alternative to MATLAB/Simulink, built specifically for the constraints of 32-bit microcontrollers such as the Infineon AURIX TriCore and ARM Cortex-M4.

| 💡 | If you have ever written a block diagram on paper and then manually translated it into C code, EmbedSim automates exactly that pipeline. |
| --- | --- |

With EmbedSim you can:

- Connect signal-processing blocks using a clean >> operator — just like Simulink's wire connections.

- Simulate the resulting system in Python with float32 precision, matching typical MCU arithmetic.

- Automatically emit embedsim_loop.c and embedsim_loop.h — ready-to-compile C source — with one function call.

- Gradually accelerate hot blocks using compiled Cython wrappers, without touching the rest of your Python model.

- Import Modelica/FMU plant models alongside your custom blocks through first-class FMU co-simulation support.

## EmbedSim in One Picture

The workflow follows a natural left-to-right pipeline:

| Python Model (block diagram) | → | Simulation (float32, RK4) | → | Code Generator (embedsim_gen/) | → | MCU Target (AURIX / Cortex-M4) | → | ASIL-D Compliant C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

# 2  Motivation — Why EmbedSim Exists

## The Problem with Today's Toolchains

Embedded control engineers working on motor drives, power converters, or automotive ECUs face a painful choice:

| Approach | What's Good | What's Painful |
| --- | --- | --- |
| MATLAB / Simulink | Industry standard, huge library, autocoding | Expensive licence, closed ecosystem, black-box codegen |
| Hand-written C | Full control, zero overhead | No simulation, error-prone, slow iteration |
| Pure Python (scipy) | Free, flexible, great for analysis | Cannot generate MCU code, float64 everywhere, no block-diagram structure |
| EmbedSim | Free & open-source, block diagrams in Python, MCU-grade codegen | Younger ecosystem, community-driven |

## Design Principles

EmbedSim was designed around five non-negotiable principles that every line of the framework upholds:

### MCU Parity First

Default precision is float32 — the native type on most 32-bit controllers. Float64 is opt-in, not the default. This means simulation results match MCU behaviour by design, not by accident.

### Dual Backend from Day One

Every block runs in pure Python for fast development and testing, and can be switched to a compiled Cython/C backend for production benchmarking — with a single flag at construction time.

### Block Diagram as Code

The >> connection operator turns a Python script into an executable block diagram. Topology is explicit, readable, and version-controllable — no hidden wires in a proprietary binary.

### Code Generation, Not an Afterthought

C code emission is a first-class feature. The code generator walks the same block graph used for simulation, ensuring that what you simulate is exactly what runs on hardware.

### Open & Affordable

MATLAB Simulink with Embedded Coder costs thousands of dollars per seat per year. EmbedSim is MIT-licensed. An engineer in Colombo or Jaffna can use the full toolchain on a laptop.

# 3  Conceptual Architecture

EmbedSim is organised into four conceptual layers. Each layer has a single responsibility, and they compose cleanly — you can use layer 1 alone for pure Python simulation, or add all four layers for full MCU code generation.

| # | Layer | Responsibility |
| --- | --- | --- |
| 1 | Signal Layer | VectorSignal — the float32 data packet flowing between blocks. |
| 2 | Block Layer | VectorBlock — the universal base class. All processing, dynamics, sources, and sinks inherit from here. |
| 3 | Simulation Layer | EmbedSim — the topological executor. Discovers block order, drives time, integrates state. |
| 4 | CodeGen Layer | LoopGenerator + PYXInspector — walks the graph, inspects .pyx files, emits embedsim_loop.c/.h. |

## Layer 1 — The Signal: VectorSignal

A VectorSignal is simply a named NumPy array. It is the currency that blocks exchange. Wrapping the array in a named object gives every value a human-readable label for debugging and plotting.

from embedsim.core_blocks import VectorSignal

# A 3-phase current measurement (float32 by default)

i_abc = VectorSignal([2.1, -1.0, -1.1], name='i_abc')

# Double precision override for a precision-critical calculation

from embedsim.core_blocks import VectorSignal

angle = VectorSignal([0.0], name='theta', dtype=np.float64)

## Layer 2 — The Block: VectorBlock

VectorBlock is the universal base class. Every element in a simulation — sources, gains, integrators, Clarke transforms, PI controllers, FMU wrappers, sinks — is a VectorBlock subclass.

A block exposes exactly three methods that subclasses override:

| Method | Purpose |
| --- | --- |
| compute_py(t, dt, inputs) | Pure Python logic — always implement this first. |
| compute_c(t, dt, inputs) | Cython/C accelerated path — implement once performance matters. |
| _load_wrapper() | Imports the compiled .pyx extension into self._wrapper. |

The >> operator wires blocks together in a single line:

# Connect a sine source → Clarke transform → Park transform → PI controller → sink

source >> clarke >> park >> pi_ctrl >> sink

# Fan-out: one source feeds two blocks

source >> clarke

source >> observer

## Layer 3 — The Simulation Engine: EmbedSim

EmbedSim takes a list of sink blocks and discovers the full execution graph by walking backwards through input connections. It then:

- Runs a Depth-First Search to establish a topologically sorted execution order.

- Detects and breaks algebraic loops using VectorDelayEnhanced / LoopBreaker blocks.

- Steps forward in time using either Euler or RK4 integration.

- Calls each block's compute() in the correct order at every time step.

from embedsim import EmbedSim

sim = EmbedSim(sinks=[sink], dt=1e-4, t_end=0.1, solver='rk4')

sim.run()

## Layer 4 — Code Generation: LoopGenerator & PYXInspector

When you are ready to deploy to hardware, CodeGenerator walks the same block graph and emits C source files. Two components collaborate:

### PYXInspector

A static analyser that reads .pyx Cython wrapper files and extracts the C function signatures — number of inputs, number of outputs, required C source files. It populates block metadata (NUM_INPUTS, OUTPUT_SIZE, C_SOURCES, C_HEADERS) automatically via the __init_subclass__ hook in VectorBlock. No manual annotation required.

### LoopGenerator

Walks the topologically sorted block list and calls _emit_block() for each block, producing a single embedsim_loop.c file containing a step() function that mirrors exactly what the Python simulation does — including register (state) variables, input wiring, and output forwarding.

from embedsim.code_generator import CodeGenerator

gen = CodeGenerator(sinks=[sink])

gen.generate_loop()      # writes embedsim_gen/embedsim_loop.c

#        embedsim_gen/embedsim_loop.h

## How the Layers Interact — A Complete Example

The following sketch shows how all four layers combine for a simple gain-and-sum simulation:

import numpy as np

from embedsim.core_blocks  import VectorSignal

from embedsim.source_blocks import SineSource

from embedsim.processing_blocks import VectorGain, VectorSum

from embedsim.dynamic_blocks import VectorEnd

from embedsim import EmbedSim

from embedsim.code_generator import CodeGenerator

# ── Layer 2: define blocks ──────────────────────────────────────────────

source = SineSource('sine',  freq=50.0, amplitude=1.0)

gain   = VectorGain('K',     gain=2.0)

summer = VectorSum('add',    signs=[1, -1])

sink   = VectorEnd('output')

# ── Layer 2: wire the diagram with >> ───────────────────────────────────

source >> gain >> summer

source >> summer          # second input to summer

summer >> sink

# ── Layer 3: simulate ────────────────────────────────────────────────────

sim = EmbedSim(sinks=[sink], dt=1e-4, t_end=0.02, solver='euler')

sim.run()

# ── Layer 4: generate C code ─────────────────────────────────────────────

gen = CodeGenerator(sinks=[sink])

gen.generate_loop()    # → embedsim_gen/embedsim_loop.c

# 4  Key Abstractions — Quick Reference

| Concept | One-liner |
| --- | --- |
| VectorSignal | A named float32 NumPy array — the data packet between blocks. |
| VectorBlock | Abstract base class for every element in the diagram. |
| compute_py() | Your Python-side block logic. Always implement this first. |
| compute_c() | Cython-accelerated path. Flip use_c_backend=True to activate. |
| >> operator | Connects block A's output to block B's input list. |
| EmbedSim | The time-stepping engine. Owns topology sort, RK4, loop detection. |
| VectorEnd | Sink block. Records a signal's time history for plotting. |
| CodeGenerator | Emits embedsim_loop.c / .h from the same graph used for simulation. |
| PYXInspector | Parses .pyx files to auto-populate C metadata on block classes. |
| PYX_FILE | Class attribute pointing to a block's .pyx wrapper. Triggers auto-inspection. |
| C_INPUT_MAP | Dict mapping input port indices to named C register variables. |
| use_c_backend | Constructor flag: False = Python (default), True = compiled C. |
| DEFAULT_DTYPE | Module-level dtype (float32). Change once to switch the whole framework. |

| 📌 | Tip for beginners: start with use_c_backend=False and DEFAULT_DTYPE=np.float32. Only add C backends after you have verified the Python model is correct. |
| --- | --- |

# 5  Next Steps

Now that you understand what EmbedSim is and how it is structured, here is a recommended learning path:

### Step 1 — Run the PMSM FOC simulation

Open pmsm_foc_pwm_smc.py and run it. Watch the motor speed converge to the reference. Plot the outputs with plot_helper.

### Step 2 — Trace one block end-to-end

Pick VectorGain. Read compute_py(). Connect it to a SineSource and a VectorEnd. Run a 1-second simulation. Verify the output is 2× the input.

### Step 3 — Read the topology printer

Call topology_printer.print_topology(sinks=[...]) on your small model. See how EmbedSim discovers and labels the block graph.

### Step 4 — Generate C code

On your small model, call CodeGenerator(...).generate_loop(). Open embedsim_gen/embedsim_loop.c and read the step() function line by line.

### Step 5 — Add a new block

Write a VectorSaturation wrapper subclass with a PYX_FILE. Add C_INPUT_MAP and watch PYXInspector populate NUM_INPUTS automatically.

### Step 6 — Build for hardware

Take the generated .c and .h files, drop them into your AURIX or Cortex-M4 project, wire to your ADC/PWM ISR, and compile with TASKING or GCC.

| 🚀 | EmbedSim is designed so that each layer can be adopted independently. You do not need to use code generation to benefit from block-diagram simulation, and you do not need to use Cython to benefit from code generation. |
| --- | --- |